Guidelines for Selecting

BDE SPECIAL PROVISIONS

For the November 7, 2025 Lettings

| Designer |  |  | FAP: |  |
| --- | --- | --- | --- | --- |
| Contract No.: |  |  | Section: |  |
|  |  |  | County: |  |

| File Name | # |  | Special Provision Title | Effective | Revised | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 80099. | 1 |  | Accessible Pedestrian Signals (APS) | Apr. 1, 2003 | Jan. 1, 2022 | Do not use the BDE APS special provision. Traffic has a new D1 APS special provision that should be used instead. |
| 80274 | 2 |  | Aggregate Subgrade Improvement | Apr. 1, 2012 | Apr. 1, 2022 | Use with Aggregate Subgrade pay item for subbase and for undercuts. ADD the following note to the GENERAL NOTES on the contract plans when Aggregate Subgrade Improvement 12” (# 303001120) Pay Item is used: “THE AGGREGATE GRADATION FOR THE AGGREGATE SUBGRADE IMPROVEMENT 12” LOWER LIFT SHALL BE CS 1 OR RR 1”. If variance is needed, then use Subbase Granular Material Type B (need D1 Bureau of Materials/Geotech approval). The typical Aggregate Subgrade Improvement thickness to be used is 12”. The D1 Bureau of Material’s Geotechnical Engineer will advise Bureau of Design when to use other thicknesses and at what plan locations (e.g. in areas where there is removal of unsuitable material, etc.). The typical unit of measurement to be used is SQ YD. The CU YD unit of measurement is to be used when recommended by the D1 Bureau of Material’s Geotechnical Engineer in areas of variable depth. The CU YD pay item is typically used for undercuts only. Refer to the Geotechnical Report for guidance on undercuts. All widening and reconstruction projects should consider adding nominal quantity for undercuts (CU YD) even when not specified in Geotech Report. Do not use Aggregate Subgrade Improvement, 4” in District 1; special provision requirements cannot be met at this thickness. Use Subbase Granular Material, Type B or consult D1 Bureau of Material's Geotech Engineer. |
| 80192 | 3 |  | Automated Flagger Assistance Devices | Jan. 1, 2008 | Apr. 1, 2023 | Use on two-lane highways where two-way traffic will be maintained over one lane of pavement in segments where no sideroads or entrances require deployment of additional flaggers. Applications include rural milling and/or resurfacing projects, bridge maintenance projects, haul road crossings, pavement patching, or other similar projects with slow moving or stationary operations where the use of a flagger is required. AFADs should not be used on projects with numerous intersections where additional flaggers are required to control traffic. |
| 80173 | 4 |  | Bituminous Materials Cost Adjustments | Nov. 2, 2006 | Aug. 1, 2017 | Include into projects with at least 1,200 tons (1,100 metric tons) of applicable bituminous work (permanent and temporary HMA mixtures, bituminous surface treatments (cover and seal coats), and pavement preservation type surface treatments. |
| 80426. | 5 |  | Bituminous Surface Treatment With Fog Seal | Jan. 1, 2020 | Jan. 1, 2022 | Insert in lieu of Check Sheet #25 for contracts involving bituminous surface treatment (preventive maintenance). |
| 80241 | 6 |  | Bridge Demolition Debris | Jul. 1, 2009 |  | Use only if a unit of local government has expressed interest in the bridge debris. The written request must be received within 16 weeks of the letting. |
| 5053I | 7 |  | Building Removal | Sep. 1, 1990 | Aug. 1, 2022 | Insert into all contracts involving building removal with no asbestos present in any of the buildings. If even one building to be removed contains asbestos, BDE Special Provision, “Building Removal with Asbestos Abatement” should be inserted instead |
| 5026I | 8 |  | Building Removal with Asbestos Abatement | Sep. 1, 1990 | Aug. 1, 2022 | Insert into all contracts involving building removal with friable and/or non-friable asbestos. This special should be used when you have multiple buildings to be removed and at least one has asbestos. Designer Note. Include the following from the asbestos containing building material (ACBM) building inspection report: (1) Sketches indicating the location of ACBMs, (2) Materials Description Table for a brief description and location of the various materials, and (3) Materials Quantities Table listing the approximate quantity of each friable and/or non-friable ACBM. In identifying the buildings (on page 1 of this BDE Special Provision), include if asbestos has been found within the description. |
| 80460 | 9 |  | Cement, Finely Divided Minerals, Admixtures, Concrete, and Mortar | Jan. 1 2025 |  | Insert into all contracts with PCC. |
| 80384. | 10 |  | Compensable Delay Costs | Jun. 2, 2017 | Apr. 1, 2019 | Insert into all contracts. |
| 80198 | 11 |  | Completion Date (Via Calendar Days) | Apr. 1, 2008 |  | Use at the District's discretion. |
| 80199 | 12 |  | Completion Date (Via Calendar Days) Plus Working Days | Apr. 1, 2008 |  | Use at the District's discretion. |
| 80461 | 13 |  | Concrete Barrier | Jan. 1, 2025 |  | Insert into contracts with CONCRETE BARRIER |
| 80453 | 14 |  | Concrete Sealer | Nov. 1, 2023 |  | This special provision should be inserted into contracts utilizing the pay item CONCRETE SEALER. |
| 80261 | 15 |  | Construction Air Quality-Diesel Retrofit | Jun. 1, 2010 | Jan. 1, 2025 | Insert into all projects. |
| 80029. | 16 |  | Disadvantaged Business Enterprise Participation | Sep. 1, 2000 | Jan. 2, 2025 | Insert into all contracts. The central office will insert the percentages of DBE required for the project. |
| 80467 | 17 |  | Erosion Control Blanket | Aug. 1, 2025 |  | This special provision should be inserted into contracts containing any erosion control blanket (temporary, permanent, or wildlife friendly). Designer Note: Designers should use the wildlife friendly erosion control blanket pay item(s) if requested by the Bureau of Maintenance – Roadside Development Unit or if there is a commitment to do so in the Phase I Natural Resource Review (NRR) Memorandum. |
| 80229 | 18 |  | Fuel Cost Adjustment | Apr. 1, 2009 | Aug. 1, 2017 | Include in projects with at least 25,000 cu yd (20,000 cu m) of earthwork items; 5,000 tons (4,500 metric tons) of applicable aggregate or hot-mix asphalt (HMA) bases course, pavement and shoulder items; 7,500 sq yd (6,000 sq m) of applicable PCC bases course, pavement and shoulder items. |
| 80452 | 19 |  | Full Lane Sealant Waterproofing System | Nov. 1, 2023 |  | Insert into contracts utilizing FULL LANE SEALANT WATERPROOFING SYSTEM. Must include pay items described in the Basis of Payment. HMA mixes must be shown on the HMA Mixture Requirements table on the plans. Consult Bureau of Materials for guidance. |
| 80447 | 20 |  | Grading and Shaping Ditches | Jan. 1, 2023 |  | Insert into all contracts using the pay item “GRADING AND SHAPING DITCHES.” |
| 80433 | 21 |  | Green Preformed Thermoplastic Pavement Markings | Jan. 1, 2021 | Jan. 1, 2022 | Insert into contracts using green thermoplastic pavement markings as part of an intersection with specific bicycle accommodation design. |
| 80471 | 22 |  | Guardrail | Nov. 1, 2025 |  | Insert into all contracts with guardrail installation, repair, or replacement. |
| 80472 | 23 |  | High Friction Surface Treatment | Nov. 1, 2025 |  | Insert into contracts utilizing HIGH FRICTION SURFACE TREATMENT. |
| 80456. | 24 |  | Hot-Mix Asphalt | Jan. 1, 2024 | Jan. 1, 2025 | Do not use in D-1, instead parts of the special the district is able to follow are contained in the HMA BINDER AND SURFACE COURSE (D-1) that must be inserted into all contracts with HMA. |
| 80446 | 25 |  | Hot-Mix Asphalt - Longitudinal Joint Sealant | Nov. 1, 2022 | Aug. 1, 2023 | Use on ALL contracts containing the LONGITUDINAL JOINT SEALANT pay item. |
| 80438. | 26 |  | Illinois Works Apprenticeship Initiative – State Funded Contracts | Jun. 2, 2021 | Apr. 2, 2024 | Insert into all nonfederal-aid contracts. |
| 80450 | 27 |  | Mechanically Stabilized Earth Retaining Walls | Aug. 1, 2023 | Aug. 1, 2025 | Insert into contracts requiring permanent or temporary mechanically stabilized earth (MSE) retaining walls. |
| 80464 | 28 |  | Pavement Marking | Apr. 1, 2025 | Nov. 1, 2025 | Insert into all contracts with pavement markings. |
| 80468 | 29 |  | Pavement Patching | Aug. 1, 2025 |  | This special provision should be inserted into contracts containing pavement patching. |
| 80441 | 30 |  | Performance Graded Asphalt Binder | Jan. 1, 2023 |  | Inset into all contracts with HMA mixes. |
| 80459 | 31 |  | Preformed Plastic Pavement Marking | June 2, 2024 |  | Insert into all contracts with Preformed Plastic Pavement Marking pay items. |
| 3426I. | 32 |  | Railroad Protective Liability Insurance | Dec. 1, 1986 | Jan. 1, 2022 | Insert into all contracts with railroads. Check with Railroad Unit to complete the Special Provision. |
| 80473 | 33 |  | Raised Reflective Pavement Markers | Nov. 1, 2025 |  | Insert into all contracts with raised reflective pavement markers. |
| 80455 | 34 |  | Removal and Disposal of Regulated Substances | Jan. 1. 2024 | Apr. 1, 2024 | Insert into all construction contracts. NOTE: This BDE special provision will be inserted into all construction contracts regardless of excavation. If there is known excavation, insert the BDE version of this special provision and also insert the District specific version provided by RSU. Both special provisions will be included in the special provisions and kept separate. |
| 80474 | 35 |  | Residential Driveway Temporary Signal | Nov. 1, 2025 |  | Use with Highway Standards 701316 and 701321 on two-lane two-way roadways with residential driveways in the work zone where two-way traffic will be maintained over one lane of pavement and controlled by a temporary bridge traffic signal system. |
| 80445 | 36 |  | Seeding | Nov. 1, 2022 |  | This special provision should be inserted into all contracts with SEEDING or INTERSEEDING. |
| 80457 | 37 |  | Short Term and Temporary Pavement Markings | Apr. 1, 2024 | Apr. 2, 2024 | Insert into all contracts with short term or temporary pavement markings. The following General Note will need to be discontinued from all projects that involve roadway resurfacing to read “type IV tape” in lieu of “type III tape” which is being discontinued. “Pavement marking tape, type IV shall be used for short term pavement markings on all final surfaces.” Designers will also need to discontinue adding pay items for temporary pavement markings type III tape for any project where temporary pavement marking tapes are being specified. |
| 80462 | 38 |  | Sign Panels and Appurtenances | Jan. 1, 2025 | Apr. 1, 2025 | Insert into contracts with SIGN PANELS. |
| 80469 | 39 |  | Slope Wall | Aug. 1, 2025 |  | This special provision should be inserted into contracts containing SLOPE WALL. |
| 80448 | 40 |  | Source of Supply and Quality Requirements | Jan. 2, 2023 |  | Insert into all Federal-aid contracts. |
| 80340 | 41 |  | Speed Display Trailer | Apr. 2, 2014 | Jan. 1, 2022 | Do not use with Standard 701400; Use the D1 version instead with Standard 701400. However, include this BDE version in contracts that include Speed Display Trailers on arterial routes ONLY (when Standard 701400 is NOT included), and include pay item for SPEED DISPLAY TRAILER. |
| 80127 | 42 |  | Steel Cost Adjustment | Apr. 2, 2004 | Nov. 1, 2025 | Include in all projects involving steel metal piling (excluding temporary sheet piling), structural steel, and reinforcing steel. |
| 80397. | 43 |  | Subcontractor And DBE Payment Reporting | Apr. 2, 2018 |  | Insert into all contracts. |
| 80391. | 44 |  | Subcontractor Mobilization Payments | Nov. 2, 2017 | Apr. 1, 2019 | Insert into all contracts. |
| 80463 | 45 |  | Submission of Bidders List Information | Jan. 1, 2025 | Mar. 2, 2025 | Insert into all contracts. |
| 80437 | 46 |  | Submission of Payroll Records | Apr. 1, 2021 | Nov. 2, 2023 | This special provision should be inserted into federal and nonfederal aid contracts on the state letting. |
| 80435 | 47 |  | Surface Testing of Pavements – IRI | Jan. 1, 2021 | Jan. 1, 2023 | Inset into all contracts involving new concrete pavement, PCC overlays, full-depth HMA, and HMA overlays with at least 2.25 in. total thickness of new HMA combined with either HMA binder or HMA surface removal. |
| 80465 | 48 |  | Surveying Services | Apr. 1, 2025 |  | Insert into all contracts. |
| 80466 | 49 |  | Temporary Rumble Strips | Apr. 1, 2025 |  | Insert into contracts with TEMPORARY RUMBLE STRIPS. |
| 80470 | 50 |  | Traffic Signal Backplate | Aug. 1, 2025 |  | Do not use in D1. Use D1 version instead. |
| 20338 | 51 |  | Training Special Provisions | Oct. 15, 1975 | Sept. 2, 2021 | Insert in all Federal Funded Projects |
| 80429. | 52 |  | Ultra-Thin Bonded Wearing Course | Apr. 1, 2020 | Jan. 1, 2022 | To be inserted as directed by the Bureau of Materials. |
| 80439 | 53 |  | Vehicle and Equipment Warning Lights | Nov. 1, 2021 | Nov. 1, 2022 | This special provision should be inserted into contracts with closed lane construction. It should not be used with moving operations only contracts. |
| 80458 | 54 |  | Waterproofing Membrane System | Aug. 1, 2024 |  | Insert into contracts using the pay item Waterproofing Membrane System. |
| 80302 | 55 |  | Weekly DBE Trucking Reports | Jun. 2, 2012 | Jan. 2, 2025 | Insert into all contracts. |
| 80454 | 56 |  | Wood Sign Support | Nov. 1, 2023 |  | This special provision should be inserted into contracts utilizing the pay item WOOD SIGN SUPPORT. |
| 80427. | 57 |  | Work Zone Traffic Control Devices | Mar. 2, 2020 | Jan. 1, 2025 | Insert into all contracts. |
| 80071 | 58 |  | Working Days | Jan. 1, 2002 |  | Use in all Working Day Contracts. Requires entry of the calculated number of working days. (15 min., round to nearest 5 days) |