# 4W

# HOT-MIX ASPHALT SURFACE REMOVAL (DECK)

(Effective April 5, 2024)

This item shall consist of furnishing all labor and equipment for the removal and satisfactory disposal of up to 1 ½” of hot-mix asphalt surface from the bridge deck area as shown on the plans.

Removal of up to 1 ½” of hot-mix asphalt surface from the bridge deck shall be according to the applicable portions of Section 440 of the Standard Specifications.

Any damage done to the concrete deck shall be corrected at the Contractor’s expense. The Contractor shall exercise care during removal of the hot-mix asphalt as to not damage the waterproofing/sand seal layer. If the waterproofing/sand seal layer is damaged due to the Contractor’s removal or paving operations, the waterproofing/sand seal shall be repaired to the satisfaction of the Engineer at the Contractor’s expense. If waterproofing/sand seal deterioration or damage is found not due to the Contractor’s removal operations, the waterproofing/sand seal shall be repaired to the satisfaction of the Engineer and shall be paid for according to Article 109.04 of the Standard Specifications.

Method of Measurement.

This work will be measured for payment as follows:

- Contract Quantities.  The requirement for use of contract quantities shall be according to Article 202.07(a) of the Standard Specifications.

- Measured Quantities.  This work will be measured for payment and the area of hot-mix asphalt removed shall be calculated in square yards.

Basis of Payment.  This work, as herein specified, will be paid for at the contract unit price per square yard for HOT-MIX ASPHALT SURFACE REMOVAL (DECK), which price shall include removal of all hot-mix asphalt surfaces.