# SECTION CORNER MARKERS

Effective:  April 15, 2006 Rev. 7-9-2020

Description.  This work shall consist of resetting section corner markers and reference markers that are disturbed prior to or during construction.

CONSTRUCTION REQUIREMENTS

General.  An Illinois Professional Land Surveyor shall be retained by the Contractor to set reference markers and section corner markers.  Monument records of the section corners shall be filed with the _______________ County Recorder of Deeds in accordance with the Land Survey Monuments Act (765 ILCS 220/0.01 et seq) of the Revised Illinois Statutes.

Section Corner Markers.  The section corner markers shall consist of a Type I aluminum tablet with magnet as shown on Highway Standard 667101, except as modified by the detail.  Said corners shall be set after the construction work is complete, and there is no possibility of disturbance of the corner.  Section corners shall be set in accordance with the Land Survey Monuments Act (765 ILCS 220/0.01 et seq) of the Revised Illinois Statutes and as prescribed by U. S. Public Act 79-649.

Reference Markers.  Reference markers shall be set clear of proposed ditch bottoms, side slopes, back slopes, and utility lines.

Method of Measurement.  Resetting of markers that are disturbed through no fault of the Contractor will be measured for payment as each.  Resetting of markers that are not protected and carefully preserved according to Article 107.20 of the Standard Specifications will not be measured for payment.

Basis of Payment.  This work will be paid for at the contract unit price per each for SECTION CORNER MARKERS.