# TRAFFIC SIGNAL GENERAL REQUIREMENTS

Effective: May 22, 2002

Revised: March 1, 2024

800.01TS

These Traffic Signal Special Provisions and the "District One Standard Traffic Signal Design Details” supplement the requirements of the State of Illinois “Standard Specifications for Road and Bridge Construction.” The intent of these Special Provisions is to prescribe the materials and construction methods commonly used for traffic signal installations.

All material furnished shall be new unless otherwise noted herein. Traffic signal construction and maintenance work shall be performed by personnel holding current International Municipal Signal Association (IMSA)/Illinois Public Service Institute (IPSI) Traffic Signal Technician Level II certification. A copy of the certification shall be immediately available upon request of the Engineer. The work to be done under the Contract consists of furnishing, installing, and maintaining all traffic signal work and items as specified in the plans and as specified herein in a manner acceptable and approved by the Engineer.

Definitions of Terms.

Add the following to Section 101 of the Standard Specifications:

101.56    Manufacturer.  Company that sells a particular type of product directly to the Contractor or the Vendor.

101.57   Vendor.  Company that supplies, represents, and provides technical support for IDOT District One approved traffic signal controllers and other related equipment.  The Vendor shall be located within IDOT District One and shall:

- Be full service with on-site facilities to assemble, test and troubleshoot traffic signal controllers and cabinet assemblies.

- Maintain an inventory of IDOT District One approved controllers and cabinets.

- Be staffed with permanent sales and technical personnel able to provide traffic signal controller and cabinet expertise and support.

- Have technical staff that hold current IMSA/IPSI Traffic Signal Technician Level III certification and shall attend traffic signal turn-ons as well as cabinet and/or controller modifications.

Submittals.

Revise Article 801.05 of the Standard Specifications to read:

“All material approval requests shall be submitted electronically following District guidelines unless directed otherwise by the Engineer.  Submittal requirements shall include, but not limited to the following:

- All material approval requests shall be made prior to or no later than the date of the preconstruction meeting.  A list of major traffic signal items can be found in Article 801.05.  Material or equipment which is similar or identical shall be the product of the same manufacturer, unless necessary for system continuity.  Traffic signal materials and equipment shall bear the U.L. label whenever such labeling is available.

- Product data and shop drawings shall be assembled by pay item.  Only the top sheet of each pay item submittal will be stamped by the Department with the review status, except shop drawings for mast arm pole assemblies and the like will be stamped with the review status on each sheet.

- Original manufacturer published product data and shop drawing sheets with legible dimensions and details shall be submitted for review.

- When hard copy submittals are necessary, four (4) complete copies of the manufacturer’s descriptive literatures and technical data for the traffic signal materials shall be submitted.  For hard copy or electronic submittals, the descriptive literature and technical data shall be adequate for determining whether the materials meet the requirements of the plans and specifications.  If the literature contains more than one item, the Contractor shall indicate which item or items will be furnished.

- When hard copy submittals are necessary for structural elements, four (4) complete copies of the shop drawings for the mast arm assemblies and poles, and the combination mast arm assemblies and poles showing, in detail, the fabrication thereof and the certified mill analyses of the materials used in the fabrication, anchor rods, and reinforcing materials shall be submitted.

- Partial or incomplete submittals will be returned without review.

- Certain non-standard mast arm poles and special structural elements will require additional review from IDOT’s Central Office.  Examples include ornamental/decorative, non-standard length mast arm pole assemblies and monotube structures.

- The Contract number or Permit number, project location/limits, and corresponding pay code number must be on each sheet of correspondence, material approval, and mast arm poles and assemblies drawings.

- Where certifications and/or warranties are specified, the information submitted for approval shall include certifications and warranties.  Certifications involving inspections and/or tests of material shall be complete with all test data, dates, and times.

- After the Engineer reviews the submittals for conformance with the design concept of the project, the Engineer will stamp the drawings indicating their status as ‘Approved’, ‘Approved-As-Noted’, ‘Disapproved’, or ‘Incomplete’.  Since the Engineer's review is for conformance with the design concept only, it is the Contractor's responsibility to coordinate the various items into a working system as specified.  The Contractor shall not be relieved from responsibility for errors or omissions in the shop, working, layout drawings, or other documents by the Department's approval thereof.  The Contractor must still be in full compliance with Contract and specification requirements.

- The Contractor shall secure approved materials in a timely manner to assure construction schedules are not delayed.

- All submitted items reviewed and marked ‘APPROVED AS NOTED’, ‘DISAPPROVED’, or ‘INCOMPLETE’ are to be resubmitted in their entirety, unless otherwise indicated within the submittal comments, with a disposition of previous comments to verify Contract compliance at no additional cost to the Contract.

- Exceptions to and deviations from the requirements of the Contract Documents will not be allowed.  It is the Contractor’s responsibility to note any deviations from Contract requirements at the time of submittal and to make any requests for deviations in writing to the Engineer.  In general, substitutions will not be acceptable.  Requests for substitutions must demonstrate that the proposed substitution is superior to the material or equipment required by the Contract Documents.  No exceptions, deviations or substitutions will be permitted without the approval of the Engineer.

- The Contractor shall not order major equipment such as mast arm assemblies prior to Engineer approval of the Contractor marked proposed traffic signal equipment locations to assure proper placement of Contract required traffic signal displays, push buttons and other facilities.  Field adjustments may require changes in proposed mast arm length and other coordination.

- Revised cabinet wiring diagrams shall be submitted whenever any wiring modifications are made to the traffic signal cabinet.”

Marking Proposed Locations.

Revise “Marking Proposed Locations for Highway Lighting System” of Article 801.09 to read “Marking Proposed Locations for Highway Lighting System and Traffic Signals.”

Add the following to Article 801.09 of the Standard Specifications:

“It shall be the Contractor's responsibility to verify all dimensions and conditions existing in the field prior to ordering materials and beginning construction.  This shall include locating the mast arm foundations and verifying the mast arms lengths.”

Inspection of Electrical Systems.

Add the following to Article 801.10 of the Standard Specifications:

- All cabinets, including temporary traffic signal cabinets, shall be assembled by an approved Vendor in District One.  The Department reserves the right to request any controller and cabinet to be tested at the Vendor’s facility prior to field installation at no extra cost to the Contract.

Maintenance and Responsibility of Traffic Signal and Flashing Beacon Installations.

Replace Article 801.11(b) of the Standard Specifications to read:

- Traffic Signals and Flashing Beacons.  The Contractor shall be responsible for maintaining the traffic signal/flashing beacon installation in proper operating condition.

- General.

  - The Contractor must notify the Area Traffic Signal Maintenance and Operations Engineer of their intent to begin any physical construction work on the Contract or any portion thereof.  This notification must be made a minimum of seven (7) working days prior to the start of construction to allow sufficient time for inspection of the existing traffic signal installation(s) and transfer of maintenance to the Contractor.  The Department will attempt to fulfill the Contractor’s inspection date request(s); however, workload and other conditions may prevent the Department from accommodating specific dates or times.  The Contractor shall not be entitled to any other compensation if the requested inspection date(s) cannot be scheduled by the Department.

  - Full maintenance responsibility shall start upon the successful completion of a maintenance transfer inspection, or as directed by the Engineer.  If the Contractor begins any physical work on the Contract or any portion thereof prior to a traffic signal inspection, maintenance of the traffic signal installation(s) will be transferred to the Contractor without an inspection.  The Contractor will become responsible for repairing or replacing all equipment that is not operating properly or is damaged at the time of transfer at no cost to the owner of the traffic signal equipment.  Final repairs or replacement of damaged equipment must meet the approval of the Engineer prior to or at the time of final inspection, otherwise the traffic signal installation will not be accepted.

  - All traffic signals within the limits of the Contract or those which have the item "MAINTENANCE OF EXISTING TRAFFIC SIGNAL INSTALLATION,” “TEMPORARY TRAFFIC SIGNAL INSTALLATION”, “TEMPORARY BRIDGE TRAFFIC SIGNAL INSTALLATION”, “TEMPORARY PORTABLE BRIDGE TRAFFIC SIGNAL INSTALLATION”, and/or “MAINTENANCE OF EXISTING FLASHING BEACON INSTALLATION” shall become the full responsibility of the Contractor.  Maintenance responsibility shall end upon issuance of final acceptance by the Engineer.

  - The Contractor shall have electricians with IMSA/IPSI Traffic Signal Technician Level II certification on staff to provide signal maintenance.  A copy of the certification shall be immediately available upon request by the Engineer.

  - This item shall include maintenance of all traffic signal equipment and other connected and related equipment such as flashing beacons, emergency vehicle preemption (EVP) equipment, master controllers, network switches, uninterruptable power supply (UPS) and batteries, pan-tilt-zoom (PTZ) cameras, vehicle detection, handholes, lighted signs, telephone service installations, cellular modems, radios, communication cables, and other traffic signal equipment.  All conduit and related equipment to adjacent intersections shall be maintained to the far back handhole, or as directed by the Engineer.  If adjacent intersections are part of Contract work, then maintenance of all conduit and related equipment shall be included in this item.

  - Regional transit, County, and other agencies may also have equipment connected to existing traffic signal or peripheral equipment such as network switches and transit signal priority (TSP, SCP, and BRT) servers, radios, and other devices, where maintenance shall be coordinated with the owner.

  - Maintenance shall not include automatic traffic enforcement equipment such as red light enforcement cameras, detectors, or peripheral equipment.  This equipment is operated and maintained by others and shall be deactivated while on Contractor maintenance.

  - The energy charges for the operation of the traffic signal installation shall be paid for by the Contractor.

- Maintenance.

  - The Contractor shall inspect all traffic signal equipment and appurtenances every two (2) weeks to ensure they are functioning properly.  Signal heads shall be properly adjusted, including plumb, and tightly mounted.  All controller cabinets, signal posts, and controller pedestals shall be tight on their foundations and in alignment.  Deficient equipment shall be repaired or replaced as necessary.  The Contractor shall check signal system communications and phone lines to assure proper operation.  This item includes, as routine maintenance, all portions of EVP equipment.  The Contractor shall always maintain enough materials and equipment in stock to provide effective temporary and permanent repairs.  The Contractor shall supply a detailed maintenance log monthly that includes dates, locations, names of electricians performing the required checks and inspections, and any other information requested by the Engineer.  The Contractor shall attend any additional inspections as requested by the Engineer.  The Contractor shall check the controllers, relays, and detectors after receiving complaints or calls to ascertain that they are functioning properly and make all necessary repairs and replacement.

  - The Contractor is advised that the existing and/or temporary traffic signal installation must remain in operation during all construction stages, except for the most essential down time.  Any shutdown of the traffic signal installation which exceeds fifteen (15) minutes must have prior approval from the Engineer.  Approval to shut down the traffic signal installation will only be granted during the period extending from 9:00 a.m. to 3:00 p.m. on weekdays.  Shutdowns shall not be allowed during inclement weather or holiday periods.

  - The Contractor shall provide immediate corrective action when any part(s) of the signal fail to function properly.  Two far side heads facing each approach shall be considered the minimum acceptable signal operation pending permanent repairs.  When repairs at a signalized intersection require that the controller be disconnected or otherwise removed from normal operation, and power is available, the Contractor shall place the traffic signal installation in flashing operation.  The signals shall flash RED for all directions unless a different indication has been specified by the Engineer.  The Contractor shall install cones on all lane lines at the stop bar on each approach, R1-1 (36 in. minimum) “STOP” signs at the stop bar on each approach on the right side and on raised medians (where applicable), and black on fluorescent orange “SIGNALS OUT AHEAD” warning signs followed by fluorescent orange W3-1 symbolic stop ahead warning signs on all approaches to the intersection.

  - Temporary replacement of a damaged or knocked down mast arm pole assembly shall require construction of a full or partial span wire signal installation or other method approved by the Engineer to assure signal heads are located overhead and over traveled pavement.  Temporary replacement of mast arm mount signals with post mount signals is not permitted.

  - The Contractor shall provide the Engineer with two (2) 24-hour telephone numbers for the maintenance of the traffic signal installation and for emergency calls by the Engineer.

  - Traffic signal equipment which is lost, damaged, or not returned to the Department for any reason shall be replaced with new equipment meeting the requirements of the Standard Specifications and these special provisions.

  - The Contractor shall be fully responsible for the safe and efficient operation of the traffic signals and other equipment noted herein.  The Contractor shall respond to all emergency calls from the Department or others within one (1) hour after notification and provide immediate corrective action.  When equipment has been damaged or becomes faulty beyond repair, the Contractor shall replace it with new equipment meeting current District One traffic signal specifications.  The cost of furnishing and installing the replaced equipment shall be borne by the Contractor at no additional cost to the Contract.  The Contractor may institute action to recover damages from a responsible third party.  If at any time the Contractor fails to perform all work as specified herein to keep the traffic signal installation in proper operating condition, or if the Engineer cannot contact the Contractor's designated personnel, the Engineer shall have the Department's Electrical Maintenance Contractor perform the maintenance work.  The Contractor shall be responsible for all of the Department’s Electrical Maintenance Contractor’s costs and liquidated damages of $1,000 per day per occurrence.  The Department's Electrical Maintenance Contractor shall bill the Contractor for the total cost of the work.  The Contractor shall pay this bill within thirty (30) days of the date of receipt of the invoice or the cost of such work will be deducted from the amount due the Contractor.  The Contractor shall allow the Electrical Maintenance Contractor to inspect the traffic signal installation that has been transferred to the Contractor for maintenance.  Final replacement of damaged equipment must meet the approval of the Engineer prior to or at the time of final inspection, otherwise the traffic signal installation will not be accepted.  Cable splices outside the controller cabinet shall not be allowed.  The Department may inspect any signalizing device on the Department’s highway system at any time without notification.  The Contractor shall not install padlocks on traffic signal cabinets or otherwise restrict the Department’s access to the cabinet or controller.

  - Any proposed activity in the vicinity of a highway-rail grade crossing must adhere to the guidelines set forth in the current edition of the Manual on Uniform Traffic Control Devices (MUTCD) regarding work in temporary traffic control zones in the vicinity of highway-rail grade crossings which states that lane restrictions, flagging, or other operations shall not create conditions where vehicles can be queued across the railroad tracks.  If the queuing of vehicles across the tracks cannot be avoided, a uniformed law enforcement officer or flagger shall be provided at the crossing to prevent vehicles from stopping on the tracks, even if automatic warning devices are in place.

  - The Contractor shall be responsible to clear snow, ice, dirt, debris, vegetation, temporary fence, or other condition that obstructs visibility of any traffic signal display or access to traffic signal equipment.

  - The Contractor shall maintain the traffic signal in normal operation during any loss of utility or battery backup power.  Temporary power to the traffic signal must meet applicable NEC and OSHA guidelines and may include portable generators and/or replacement batteries.  Temporary power shall not be paid for separately but shall be included in the Contract.

- Basis of Payment.  This work will be paid for at the Contract unit price per each for MAINTENANCE OF EXISTING TRAFFIC SIGNAL INSTALLATION, TEMPORARY TRAFFIC SIGNAL INSTALLATION, TEMPORARY BRIDGE TRAFFIC SIGNAL INSTALLATION, or TEMPORARY PORTABLE BRIDGE TRAFFIC SIGNAL INSTALLATION.  Each location will be paid for separately.  Maintenance of a flashing beacon shall be paid for at the Contract unit price for MAINTENANCE OF EXISTING FLASHING BEACON INSTALLATION.  Each flashing beacon will be paid for separately.

Damage to Traffic Signal System.

Add the following to Article 801.12(b) of the Standard Specifications:

“Any traffic signal control equipment that is damaged and non-repairable or not operating properly from any cause shall be replaced with new equipment meeting current District One traffic signal specifications and provided by the Contractor at no additional cost to the Contract and/or owner of the traffic signal system, all as approved by the Engineer.  Final replacement of damaged equipment must meet the approval of the Engineer prior to or at the time of final inspection.  Repair or replace any equipment damaged within the time shown in the table below:

| ITEM | RESPONSE TIME | SERVICE RESTORATION | PERMANENTREPAIR (calendar days) |
| --- | --- | --- | --- |
| Cabinet | 1 hour | 24 hours | 21 days |
| Controllers and Peripheral Equipment | 1 hour | 4 hours | 21 days |
| System Detector Loop | 1 hour | N/A | 7 days |
| All Other Detectors | 1 hour | N/A | 21 days |
| Signal Head and Lenses | 1 hour | 4 hours | 7 days |
| Aviation Red Beacon | 1 hour | 4 hours | 7 days |
| Mast Arm Assembly and Pole | 1 hour | 4 hours | 7 days |
| Traffic Signal Post | 1 hour | 4 hours | 7 days |
| Cable and Conduit | 1 hour | 4 hours | 7 days |
| Interconnect and Telemetry | 1 hour | 4 hours | 7 days |
| Graffiti Removal | N/A | N/A | 7 days |
| Misalignment of Signal Heads | 1 hour | 4 hours | 4 hours |
| Closed Loop Monitoring System | 1 hour | 24 hours | 14 days |
| Post and Poles Plumb Vertically | N/A | N/A | 21 days |
| Controller, Post & Pole Foundations | N/A | N/A | 21 days |
| Complaints, Calls, Controller or System Alarms, Timing, Phasing, Programming | 1 hour | 4 hours | N/A |
| Patrol Truck Deficiencies | N/A | 24 hours | 24 hours |
| Signal Heads Visibility | 1 day | 2 days | 14 days |

Temporary replacement of a damaged or knocked down mast arm pole assembly shall require construction of a full or partial span wire signal installation or other method approved by the Engineer to assure signal heads are located overhead and over traveled pavement.  Temporary replacement of mast arm mount signals with post mount signals will not be permitted.

Replacement of any equipment for any reason shall be reported to the Area Traffic Signal Maintenance and Operations Engineer in writing within 24 hours.  Permanent and temporary replacement of the controller and/or cabinet shall require inspection and testing by the Vendor.

Automatic Traffic Enforcement equipment, such as red light enforcement cameras, detectors, and peripheral equipment, that is damaged or not operating properly from any cause, shall be the responsibility of the municipality or the automatic traffic enforcement company per Permit agreement.”

Traffic Signal Inspection (TURN-ON).

Revise Article 801.15(b) of the Standard Specifications to read:

“Turn-on.  It is the intent to have all electric work completed and equipment field tested by the Contractor and/or Vendor prior to the Department’s “turn-on” field inspection.  If in the event the Engineer determines work is not complete and the inspection will require more than two (2) hours to complete, the inspection shall be canceled, and the Contractor will be required to reschedule at another date.  The maintenance of the traffic signals will not be accepted until all punch list work is corrected and re-inspected.

When the Contractor requests a turn-on and inspection of the completed traffic signal installation(s), the request must be made to the Area Traffic Signal Maintenance and Operations Engineer a minimum of seven (7) working days prior to the time of the requested inspection.  The Department will attempt to fulfill the Contractor’s turn-on and inspection date request(s); however, workload and other conditions may prevent the Department from accommodating specific dates or times.  The Contractor shall not be entitled to any other compensation if the requested turn-on and inspection date(s) cannot be scheduled by the Department.  The Department will not grant a field inspection until written or electronic notification is provided from the Contractor that the equipment has been field tested and the intersection is operating according to Contract requirements.  The Contractor must invite local fire department personnel to the turn-on when emergency vehicle preemption (EVP) is included in the project.  When the Contract includes the item RE-OPTIMIZE TRAFFIC SIGNAL SYSTEM, OPTIMIZE TRAFFIC SIGNAL SYSTEM, and/or TEMPORARY TRAFFIC SIGNAL TIMING, the Contractor must notify the SCAT Consultant of the turn-on/detour implementation schedule, as well as stage changes and phase changes during construction.

The Contractor must have all traffic signal work completed and the electrical service installation connected by the utility company prior to requesting an inspection and turn-on of the traffic signal installation.  The Contractor shall be responsible to provide a police officer to assist with traffic control at the time of testing.

The Contractor shall provide a representative from the Vendor who is knowledgeable of the cabinet design and controller functions to attend the traffic signal inspection for both permanent and temporary traffic signal turn-ons.

Upon demonstration that the signals are operating and all work is completed in accordance with the Contract and to the satisfaction of the Engineer, the Engineer will then allow the signals to be placed in continuous operation.  The signals shall continue to be maintained by the Contractor until final acceptance.

The Department requires the following Final Project Documentation from the Contractor at traffic signal turn-ons in electronic format in addition to hard copies where noted.  An electronic media device shall be submitted with separate folders corresponding to each numbered title below.  The electronic media device shall be labeled with date, project location, company, and Contract or Permit number.  Electronic record drawings and material approvals shall be submitted prior to traffic signal turn-on for review by the Department as described in the Record Drawings section herein.

Final Project Documentation:

- Record Drawings.  Electronically produced signal plans of record with field revisions marked in red.  Two (2) hard copies of 11 in. x 17 in. record drawings shall also be provided.

- Field Testing.  Written notification from the Contractor and the Vendor of satisfactory field testing with corresponding material performance measurements, such as for detector loops and fiber optic systems (see Article 801.13).

- Material Approvals.  Material approval documentation.

- Manuals.  Operation and service manuals of the signal controller and associated control equipment.

- Cabinet Wiring Diagram and Cable Logs.  Five (5) hard copies of 11 in. x 17 in. cabinet wiring diagrams shall be provided along with electronic PDF and DGN files of the cabinet wiring diagram.  Five (5) hard copies of the cable logs and electronic Excel files shall be provided with cable #, number of conductors and spares, connected device/signal head and intersection location.

- Warrantees and Guarantees.  All manufacturer and Contractor warrantees and guarantees required by Article 801.14.

- GPS Coordinates.  GPS coordinates of traffic signal equipment as described in the Record Drawings section herein.

Acceptance of the traffic signal equipment by the Department shall be based upon inspection results at the traffic signal “turn-on”, completeness of the required documentation, and successful operation during a minimum 72 hour “burn-in” period following activation of traffic signal equipment.  If approved, traffic signal acceptance shall be verbal at the final inspection followed by written correspondence from the Engineer.  The Contractor shall be responsible for all traffic signal equipment and associated maintenance thereof until Departmental acceptance is granted.

All equipment and/or parts to keep the traffic signal installation operating shall be furnished by the Contractor.  No spare traffic signal equipment is available from the Department.

All punch list work shall be completed within two (2) weeks after the turn-on.  The Contractor shall notify the Area Traffic Signal Maintenance and Operations Engineer to schedule an inspection of all punch list work.  Failure to meet these time constraints shall result in liquidated damage charges of $500 per month per incident.

All cost of work and materials required to comply with the requirements herein shall be included in the pay item bid prices, under which the subject materials and signal equipment are paid, and no additional compensation will be allowed.  Materials and signal equipment not complying with the requirements herein shall be subject to removal and disposal at the Contractor's expense.”

Record Drawings.

The requirements listed for Electrical Installation shall apply for Traffic Signal Installations in Article 801.16. Revise the second and third paragraphs of Article 801.16 of the Standard Specifications to read:

“When the work is complete, and seven (7) days before the request for a final inspection, electronic Contract drawings, stamped “RECORD DRAWINGS”, shall be submitted to the Engineer for review and approval and shall be stamped with the date and the signature of the Contractor’s supervising Engineer or electrician.  The record drawings shall be submitted in PDF format.  If the Contract consists of multiple intersections, each intersection shall be saved as an individual PDF file with TS# and location name in its file name.

In addition to the record drawings, copies of the final material approvals which have been Approved or Approved as Noted shall be submitted in PDF format.  The PDF files shall clearly indicate the pay item either by filename or PDF Table of Contents referencing the respective pay item number for multi-item PDF files.  Specific part or model numbers of items which have been selected shall be clearly visible.

The Contractor shall provide two (2) 11 in. x 17 in. hard copies of electronically produced final record drawings to be kept inside each traffic signal cabinet within project limits.”

Add the following to Article 801.16 of the Standard Specifications:

“In addition to the specified record drawings, the Contactor shall record GPS coordinates of the following traffic signal components being installed, modified or being affected in other ways by the Contract:

- All Mast Arm Poles and Posts

- Traffic Signal Wood Poles

- Railroad Bungalow

- UPS

- Handholes

- Controller Cabinets

- Communication Cabinets

- Electric Service Disconnect locations

- CCTV/PTZ Camera installations

Datum to be used shall be North American 1983.

Data shall be provided in electronic format and shall be in .csv format.  Latitude and Longitude shall be in decimal degrees with a minimum of 6 decimal places.  Each coordinate shall have the following information:

- File shall be named:  TSXXX_YY-MM-DD.csv (i.e. TS22157_24-01-01.csv)

- Each intersection shall have its own file

- Row 1 should have the location name (i.e. IL 31 @ Klausen)

- Row 2 is blank

- Row 3 is the headers for the columns

- Row 4 starts the data

- Column A (Date) – should be in the following format: MM/DD/YYYY

- Column B (Item) – as shown in the table below

- Column C (Description) – as shown in the table below

- Column D and E (GPS Data) – should be in decimal form

Examples:

| Date | Item | Description | Latitude | Longitude |
| --- | --- | --- | --- | --- |
| 01/01/2024 | MP (Mast Arm Pole) | NEQ, NB, Dual, Combination Pole | 41.580493 | -87.793378 |
| 01/01/2024 | HH (Handhole) | Heavy Duty, Fiber, Intersection, Double | 41.558532 | -87.792571 |
| 01/01/2024 | ES (Electrical Service) | Ground mount, Pole mount | 41.765532 | -87.543571 |
| 01/01/2024 | CC (Controller Cabinet) |  | 41.602248 | -87.794053 |
| 01/01/2024 | PTZ (PTZ) | NEQ extension pole | 41.593434 | -87.769876 |
| 01/01/2024 | POST (Post) |  | 41.651848 | -87.762053 |
| 01/01/2024 | MCC (Master Controller Cabinet) |  | 41.584593 | -87.793378 |
| 01/01/2024 | COMC (Communication Cabinet) |  | 41.584600 | -87.793432 |
| 01/01/2024 | BBS (Battery Backup System) |  | 41.558532 | -87.792571 |

Data collection can be made as construction progresses or can be collected after all items are installed.  If the data is unacceptable, the Contractor shall make corrections to the data collection equipment and/or process and resubmit the data for review and approval as specified.

Data shall have a minimum 1 ft accuracy after post processing.”

Restoration of Work Area.

Add the following article to Section 801 of the Standard Specifications:

“801.17    Restoration of Work Area.  Restoration of the traffic signal work area shall be included in the related pay items such as foundation, conduit, handhole, underground raceways, detector loop installation or replacement, etc.  All roadway surfaces such as shoulders, medians, sidewalks, pavement, etc. shall be replaced in kind.  All damage to mowed lawns shall be replaced with an approved sod, and all damage to unmowed fields shall be seeded.  All brick pavers disturbed in the work area shall be restored to their original configuration as directed by the Engineer.  All damaged brick pavers shall be replaced with a comparable material approved by the Engineer.

Exposed holes created from removal or relocation of traffic signal equipment shall be sealed using a zinc-plated fender washer with toggle bolt.

Restoration of the work area shall be included in the Contract without any extra compensation allowed to the Contractor.

Removal, Disposal, and Salvage of Existing Traffic Signal Equipment.

The removal, disposal, and/or salvage of existing traffic signal equipment shall become the property of the Contractor and disposed of by the Contractor outside the State’s right-of-way, unless otherwise noted.  No additional compensation shall be provided to the Contractor for removal, disposal or salvage expense for the work in the Contract.”

Bagging Signal Heads.

Light tan colored traffic and pedestrian signal reusable covers shall be used to cover dark/un-energized signal sections, visors, and retroreflective backplates. Covers shall be made of outdoor fabric with urethane coating for repelling water, have elastic fully sewn around the cover ends for a tight fit over the visor, and have a minimum of two (2) straps with buckles to secure the cover to the backplate. A center mesh strip allows viewing without removal for signal status testing purposes. Covers shall include a message indicating the signal is not in service. Pedestrian pushbuttons that are not in service shall be covered with a durable material such as described above or burlap that is secured in a weather-resistant manner. The entire housing, including the pedestrian sign, shall also be covered on the front side.

Turn-on of New Traffic Signal Installations.

The following only applies to new traffic signals at previously unsignalized locations.

The signal responsibility shall begin at the start of signal construction and shall end upon issuance of final acceptance by the Engineer. New traffic signal heads and indications may not be installed more than two (2) weeks (14 calendar days) prior to the scheduled turn-on of the traffic signal to avoid motorist confusion caused by the presence of new signal heads, even if properly covered. Unenergized signal indications shall be bagged until one (1) hour prior to the scheduled turn-on per the Bagging Signal Heads section above.

New stop bars and crosswalks on approaches that did not previously have stop control shall NOT be installed until the day of the traffic signal turn-on.

A Portable Changeable Message Sign (PCMS) must be placed two (2) weeks prior to the scheduled new traffic signal turn-on for all approaches to the intersection with the following messages:

| NEW |  | STARTING |
| --- | --- | --- |
| TRAFFIC |  | MMM ## |
| SIGNAL |  |  |

where “MMM” and “##” are the 3-character month abbreviation and day of the scheduled turn-on, respectively.

On the day of the turn-on, change messages to read:

| NEW |  | BE |
| --- | --- | --- |
| SIGNAL |  | PREPARED |
| AHEAD |  | TO STOP |

The PCMS must remain in place for two (2) weeks following the day of the turn-on.

Conflicting Stop signs shall be removed immediately at the time of the traffic signal turn-on.

Locating Underground Facilities.

Revise Section 803 to the Standard Specifications to read:

“IDOT traffic signal facilities are not part of any of the one-call locating service such as J.U.L.I.E or Digger.  If the Contract requires the maintenance services of an Electrical Contractor, the Contractor shall be responsible at their own expense for locating all existing IDOT electrical facilities, including but not limited to interconnect conduit and handholes, prior to performing any work.  A maintenance transfer is required prior to any locating work.  If this Contract does not require the maintenance services of an Electrical Contractor, the Contractor may request one free locate for existing IDOT electrical facilities from the District One Electrical Maintenance Contractor prior to the start of any work.  Additional requests will be at the expense of the Contractor.  The location of underground traffic facilities does not relieve the Contractor of their responsibility to repair any facilities damaged during construction at their expense.

The exact location of all utilities shall be field verified by the Contractor before the installation of any components of the traffic signal system.  For locations of utilities, locally owned equipment, and leased enforcement camera system facilities, the local Counties or Municipalities may need to be contacted: in the City of Chicago contact Digger at (312) 744-7000, and for all other locations contact J.U.L.I.E. at 1-800-892-0123 or 811.

The Contractor shall take whatever precautions to protect the electric cable or electric conductors in conduit from damage during location and construction operations.  If the wiring is damaged, the Contractor shall replace the entire length of cable or conductors in conduit, in a manner satisfactory to the Engineer.  Splicing below grade will not be permitted.

In the event the repairs are not made by the Contractor, the Contractor shall reimburse the Department for such repairs within sixty (60) days of receiving written notification of said damage.  Otherwise, the cost of such repairs will be deducted from monies due or which will become due the Contractor under the terms of the Contract.”

Grounding of Traffic Signal Systems

Revise Section 806 of the Standard Specifications to read:

“All traffic signal systems, equipment and appurtenances shall be properly grounded in strict conformance with the NEC.  This work shall be in accordance with IDOT’s District One Traffic Signal Design Details.

The grounding electrode system shall include a ground rod installed with each traffic signal controller concrete foundation and all mast arm and post concrete foundations.  An additional ground rod will be required at locations were measured resistance exceeds 25 ohms.  Ground rods are included in the applicable concrete foundation or service installation pay item and will not be paid for separately.

Testing shall be according to Article 801.13 (a) (4) and (5).

- The grounded conductor (neutral conductor) shall be white color coded.  This conductor shall be bonded to the equipment grounding conductor only at the Electric Service Installation.  All power cables shall include one neutral conductor of the same size.

- The equipment grounding conductor shall be green color coded.  The following is in addition to Article 801.04 of the Standard Specifications:

  - Equipment grounding conductors shall be bonded to the grounded conductor (neutral conductor) only at the Electric Service Installation.  The equipment grounding conductor is paid for separately and shall be continuous.  The Earth shall not be used as the equipment grounding conductor.

  - Equipment grounding conductors shall be bonded, using a UL Listed grounding connector, to all traffic signal mast arm poles, traffic signal posts, pedestrian posts, pull boxes, handhole frames and covers, conduits, and other metallic enclosures throughout the traffic signal wiring system, except where noted herein.  Bonding shall be made with a splice and pigtail connection, using a sized compression type copper sleeve, sealant tape, and heat-shrinkable cap.  A UL listed electrical joint compound shall be applied to all conductors’ terminations, connector threads and contact points.  Conduit grounding bushings shall be installed at all conduit terminations, including spare or empty conduits and conduit protruding from handhole walls.

  - All metallic and non-metallic raceways, including spare or empty raceways, shall have a continuous equipment grounding conductor, except raceways containing only detector loop lead-in circuits, circuits under 50 V and/or fiber optic cable will not be required to include an equipment grounding conductor.

  - Individual conductor splices in handholes shall be soldered and sealed with heat shrink.  When necessary to maintain effective equipment grounding, a full cable heat shrink shall be provided over individual conductor heat shrinks.

- The grounding electrode conductor shall be similar to the equipment grounding conductor in color coding (green) and size.  The grounding electrode conductor is used to connect the ground rod to the equipment grounding conductor and is bonded to ground rods via exothermic welding, UL listed pressure connectors, and UL listed clamps.”