# advanced transportation CONTROLLER AND CABINET

Effective: March 1, 2024

Revised: January 1, 2026

857.05TS

Description.

This work shall consist of furnishing and installing a traffic actuated solid state digital controller in an Advanced Transportation Controller (ATC) cabinet of the type specified, meeting the requirements of Section 857 of the Standard Specifications, and as modified herein, including Output Assembly, Input Assembly, Service Assembly, DC Power/Communications Bus, AC Clean Power Bus, Field Output Panel (FOP), and all necessary connections for proper operation. This specification describes the shelf mount versions of the ATC cabinet, which uses a NEMA-style cabinet with ATC subassemblies mounted either on a shelf or on the cabinet side walls.

If the intersection is part of an existing system and/or when specified in the plans, this work shall consist of furnishing and installing a(n) "_______" brand traffic actuated solid state controller.

Materials.

Add the following to Article 857.02 of the Standard Specifications:

“Controllers shall be Econolite Cobalt or Eagle/Yunex M60 unless specified otherwise on the plans or elsewhere on these specifications.  Only controllers supplied by one of the District One approved Vendors will be allowed.  The controller shall be of the most recent approved model and software version supplied by the Vendor at the time of the traffic signal TURN-ON unless specified otherwise on the plans or these specifications.  A removable controller data key shall also be provided.  The controller shall be equipped with a 2070 series controller module for proper operation in an ATC cabinet.  The controller shall prevent phases from being omitted during program changes and after all preemption events and shall inhibit simultaneous display of circular yellow and yellow arrow indications.

For integration into an Advanced Traffic Management System (ATMS) such as Centracs, Tactics, or TransSuite, the controller shall have the latest version of approved NTCIP software installed.  For operation prior to integration into an ATMS, the controller shall maintain existing communications.”

Add the following to Article 1074.03 of the Standard Specifications:

(a) The cabinet shall be designed and manufactured with materials that will allow rigid mounting.  The cabinet shall be base mounted only.  The cabinet shall not flex on its mount.  The subassemblies in the cabinet shall be removable with simple hand tools, such as a screwdriver, and without removing any other equipment.  Any components over 50 V with exposed terminals shall be protected from incidental contact per NEC requirements.  All equipment in the cabinet shall be clearly and permanently labeled.  All marker strips shall be made of material that can be easily written on using a pencil or ballpoint pen.  Marker strips shall be located immediately below the item they are to identify and must be clearly visible with the items installed.  Card guides (top and bottom) shall be provided for the plug-ins.  All circuit boards shall be conformal coated to protect the boards from moisture.  All switches shall be guarded.

Each cabinet assembly shall be tested as a complete entity under signal load.  The cabinet shall be assembled and tested by the Manufacturer or Vendor to ensure proper component integration operation.

Revise the table in Article 1074.03 (a) (1) to read:

| Type | Approx. Volume cu ft (cu m) | Police Door | Mounting | Ventilation | Remark |
| --- | --- | --- | --- | --- | --- |
| I | Reserved |  |  |  |  |
| II | 5.0 (0.14) | Yes | Post Top | Filtered Air Intake and One Thermo-Statically Controlled Fan |  |
| III | 11.5 (0.33) | Yes | Ground Mount | Filtered Air Intake and One Thermo-Statically Controlled Fan | For Traffic Actuated Controller. |
| IV | 29.0 (0.82) | Yes | Ground Mount | Filtered Air Intake and Two Thermo-Statically Controlled Fan | For Traffic Actuated Controller. Back Panel with Minimum 12 Load Switch Positions or ATC Cabinet. |
| V | 44.0 (1.25) | Yes | Ground Mount | Filtered Air Intake and Two Thermo-Statically Controlled Fan | For Traffic Actuated Controller. Back Panel with Minimum 12 Load Switch Positions or ATC Cabinet. |

Add the following to Article 1074.03 of the Standard Specifications:

(a) (1) Optionally, a “Type 4.5/Type IV Stretched” cabinet shall be provided.  This cabinet has the same dimensions as a Type IV cabinet with an additional height.  Additionally, the cabinet shall include a front and rear door with double, vented overhangs for protection from water intrusion.

(a) (2) a. The gaskets shall be permanently bonded to the cabinet.  The gaskets shall

include a polyester film to prevent the gaskets from sticking to the cabinet surface.

(a) (2) e. A rain channel shall be incorporated into the design of the main door opening to prevent liquids from entering the enclosure.  The cabinet door opening shall be a minimum of 80 percent of the front surface of the cabinet.  A stiffener plate shall be welded across the inside of the main door to prevent flexing.

(a) (2) f. The top of the cabinet shall incorporate a 1 in. slope towards the rear to prevent rain accumulation.

Revise Article 1074.03 (a) (3) paragraph “a” to read:

“Multiple Door-Stop.  The main door on the cabinet shall be equipped with a three-point latching mechanism with nylon rollers.  It shall include an automatic door stop mechanism capable of holding the door open at approximately 90°, 120°, and 180° under windy conditions.  Manual placement of the mechanism shall not be required by field personnel.”

Add the following to Article 1074.03 of the Standard Specifications:

(a) (3) b. The main door shall utilize stainless steel hardware.  The handle shall include a vertically-mounted hasp for the attachment of an optional padlock.  The handle shall not extend beyond the perimeter of the main door at any time.

(a) (3) c. The lock assembly shall be positioned so that the handle does not cause any interference with the key when opening the cabinet door.  All external fasteners shall be tamper-proof.  The police door assembly shall be flush-mounted to the main door.

(a) (3) d. The cabinet shall be supplied with a natural aluminum mill finish.  Sufficient care shall be taken in handling to ensure that scratches are minimized.  All surfaces shall be free from weld flash.  Welds shall be smooth, neatly formed, and free from cracks, blowholes, and other irregularities.  All sharp edges shall be ground smooth.

(a) (3) e. The cabinet seams shall be sealed with a tack weld on the interior of the cabinet and a double flange at the top only.  Optionally, the cabinet can be supplied as a UL listed version with double flanges all the way around, and continuous welding of all seams.

(a) (3) f. All cabinets shall be supplied with a minimum of two removable shelves manufactured from 5052-H32 aluminum.  Shelves shall be a minimum of 10 in. deep.  The shelves shall include two (2) 1U mounting positions to install 1U rack-mounted equipment.  The rack mounting holes shall be tapped for #10-32 screws.

(a) (3) g. Cabinets shall include a set of three vertical “C” channels mounted on each interior side wall of the cabinet for the purpose of mounting the cabinet components.  The channels shall accommodate spring mounted nuts.  All mounting rails shall extend 3-1/8 in. from the bottom of the cabinet.  The sidewalls shall be supplied three channels, with the rear channel located at 8 in. from the back of the cabinet.  Measured from center to center, the middle channel shall be 8-1/2 in. from the rear channel, and the front rail shall be 6 in. from the middle channel.  The rear walls shall be supplied with three saw tooth rails, with the left-most rail located at 8 in. from the left side of the cabinet.  Measured from center to center, the middle rail shall be located 9 in. from the left-most rail, and the right-most rail shall be 19-1/2 in. from the middle rail.  The saw tooth rails on back wall shall allow shelf adjustments in 1/2 in. increments.

(a) (3) h. The lower section of the cabinet shall be equipped with a louvered air entrance.  The air inlet shall be large enough to allow sufficient air flow per the rated fan capacity.  Louvers shall satisfy the NEMA rod entry test for 3R ventilated enclosures.  A removeable, disposable air filter shall be secured to the air entrance, as mentioned in 1074.03 (a) (2) paragraph “c”.

(a) (3) i. The roof of the cabinet shall incorporate an exhaust plenum with a vent screen.  Perforations in the vent screen shall be 1/8 in. x 1/2 in. rectangular slots.

(a) (3) j. Anchor bolts shall be used to properly secure the cabinet to its base.  The cabinet flange for securing the anchor bolts shall not protrude outward from the bottom of the cabinet.  Four (4) 3/4 in. x 18 in. long right-angle anchor bolts shall be provided with the cabinet.

(a) (3) k. All cabinets shall be pre-wired for a minimum of eight (8) phases of vehicular, four (4) phases of pedestrian and four (4) phases of overlap operation.

Revise Article 1074.03 (a) (4) paragraph “a” to read:

“Surge Suppressor.  The suppressor protecting the cabinet equipment shall consist of two stages: stage one which shall include a controller cabinet AC power protection and stage two which shall include AC circuit protection.”

Revise Article 1074.03 (a) (5) paragraph “a” to read:

“Signal Flash in Absence of Subassemblies.  The cabinet shall be capable of remaining in flashing operation with any of the following subassemblies removed: Input Assembly, Output Assembly, Cabinet Power Supply, and Controller.”

Remove Article 1074.03 (a) (5) paragraphs “b”, “c”, and “d”.

Replace Article 1074.03 (b) to read:

(b) ATC Cabinet Subassemblies.

(1) Output Assembly.

- The Output Assembly shall accommodate 3U plug-in cards and be mounted on the shelf.  The shelf mounting shall include mounting flanges to allow the rack to be bolted to the shelf.

- The Output Assembly shall accommodate eight (8) Model 2202 High-Density Switch Pack/Flasher Units (HDSP/FU), providing a total of 48 output channels.

- The Output Assembly shall accommodate one (1) Model 2218 Serial Interface Unit (SIU) to provide interface and control via system SB1/SB2.

- The Output Assembly shall accommodate one (1) Model 2212 Cabinet Monitor Unit (CMUip).

- The Output Assembly shall include a hinged front panel with four (4) Circuit Breakers.  The front panel shall be attached with thumb screws to allow access to the wiring.  The circuit breakers shall be rated at 5 A.  The breakers shall be Carling Technologies “B” series or approved equal.  Each breaker shall protect two (2) HDSPs.  The breakers shall be protected by a flip-up cover to protect against accidental activation.

- The Output Assembly shall utilize four (4) 24-pin connectors (Molex 39-28-8240) or eight (8) 12-pin connectors (Molex 39-28-8120) to interface the HDSP outputs to the Field Output Panel (FOP).

- All exposed AC voltage on the Output Assembly circuit boards shall be protected using a removable Lexan cover mounted on standoffs.

For a 32-channel cabinet where a second Output Assembly is used, the second Output Assembly shall meet the same requirements as the first Output Assembly, with the exception of accommodating a CMUip.

(2) Field Output Panel (FOP).

- The 16-Channel FOP shall be coupled with its respective 16-Channel Output Assembly to provide pluggable connectors for the signal output field terminals, flash programming, and flash transfer relays.

- The FOP shall house eight (8) Model 21H High-Density Flash Transfer Relays (HDFTRs) and sixteen (16) Flash Program Blocks (FPBs).  The HDFTRs and FPBs shall be provided to control and select the color (red, yellow, or dark) during flashing operation.

- HDSP Suppressors shall be provided at the field terminals for the protection of the HDSP.  These suppressors shall plug in on the back side of the FOP.

- Each HDFTR position shall be labeled with the number of its associated HDSP.  Each FPB position shall be labeled with the number of its associated channel.

- The FOP shall be provided with sixteen (16) 6-position Phoenix Contact terminal block model number 1777765 plugs and 1720615 sockets.  Each Field Terminal Block socket shall be labeled with the number of its associated channel.  Additional labels shall be provided to clearly indicate which terminals correspond to the red, yellow, and green switch pack outputs.

- The FOP shall be mounted on the sawtooth rails on the back wall and shall swing down using thumbscrews to provide access to the HDSP Suppressors and the wiring for the FOP.  The panel shall be angled to allow easy access to the field terminals.

- The FOP shall utilize eight (8) 12-pin connectors, Molex 39-28-8120, to interface the HDSP outputs to the FOP.  The cables shall be long enough to allow the FOP to swing down with no interference.

For a 32-channel cabinet where a second FOP is used, the second FOP shall meet the same requirements as the first FOP.

(3) Combination Output Assembly-Field Output Panel Unit (16-Channel Loadbay).

The Output Assembly and Field Output Panel shall be optionally available as a combined unit, similar to a NEMA loadbay.  This version is only available for a 16-channel configuration.

- The loadbay shall be mounted on the sawtooth rails at the back of the cabinet below the bottom shelf.

- The Output Assembly shall be mounted at the top of the loadbay.

- The Field Output Panel shall be mounted below the Output Assembly on an angled plate to allow easy access to the field terminals.

- All other requirements for the Output Assembly shall meet Article 1074.03 (b) (1).

- All other requirements for the Field Output Panel shall meet Article 1074.03 (b) (2).

- All Field Output cables shall have electric circuit protection to prevent accidental overloads and short circuits caused in Field Outputs. The circuit breakers shall be rated to maximum of 2 amps.

(4) Input Assembly.

- The Input Assembly shall be a 3U high shelf mounted assembly providing twelve (12) slots of 22/44 pin PCB sockets for utilizing input devices such as loop detectors, AC and DC isolators, and emergency vehicle preemption (EVP) equipment.

- The Input Assembly shall accommodate one (1) Model 2218 Serial Interface Unit (SIU) to provide interface and control between the controller and the input devices via system SB1/SB2.

- The Input Assembly shall house twelve (12) 2-channel detection modules, six (6) 4-channel detection modules, or a combination of 2 & 4 channel detection modules up to 24 channels.

- The Input Assembly shall utilize two (2) 26-position ribbon cable connectors, TE Connectivity model 102321-6, for connections to the Field Input Panel.

(5) Field Input Panel (FIP).

- The 24-Channel FIP shall be coupled with the 24-Channel Input Assembly.  The FIP shall provide a convenient landing point with pluggable terminal blocks for the field input wires.

- The FIP shall have positions for landing twenty-four (24) two-wire inputs and their associated earth ground wires.  The landing points shall be pluggable terminal blocks with 10 positions for each block.  The terminal blocks shall be Phoenix Contact model 1757093 or approved equivalent.

- Each input terminal position shall be labeled with its associated channel number.

- The FIP shall have positions for twelve (12) pluggable Detection Module Suppressors.  The Detection Module Suppressors shall be supplied with the cabinet if required by procurement.

- The circuit board shall be mounted on a 1/8 in. thick aluminum plate that includes mounting slots for channel mounting.  The plate shall include pressed-in PEM standoffs to mount the circuit boards.

(6) Service Assembly.

The Service Assembly shall be modular and be accessible without the use of hand tools.  The Service Assembly shall house the following items:

- Two (2) Model 2202-HV High-Density Switch Pack/Flasher Units (HDSPs/FUs).

- Pluggable Cabinet Suppressor/Filter – Hesco RLS model HE1750 or approved equal.

- Main Contactor.

- Four (4) HDFU output fuses (16-channel version) or eight (8) HDFU output fuses (32-channel version).

- Six (6) Circuit Breakers.  The circuit breakers shall be Carling Technologies “B” series or approved equal.  The circuit breakers shall be protected by flip-up covers to protect against accidental activation. The circuit breakers shall be provided for the following functions:

  - Main – 30 A

  - Clean AC Power – 15 A

  - Raw AC Power – 15 A

  - Output Assembly – 15 A

  - HDFU #1 – Maximum 15 A

  - HDFU #2 – Maximum 15 A (optional, for 32-channel version only)

- GFCI duplex receptacle.

- Earth Ground and AC Neutral bus bars.

- Raw AC utility power input terminal block having five screw terminals.

(7) DC Power/Communication Bus.

- The DC Power/Communication Bus shall include eight (8) DB25 D-sub socket connectors, TE Connectivity model 5745886-2, to interconnect the SB1/SB2 communication ports of the assemblies and Controller.  It shall include a termination circuit at the end of the connections (S8) to prevent radio frequency signal reflection.

- The DC Power/Communication Bus shall include eight Phoenix Contact connectors, model 1830635, to bring DC power to the Bus and distribute it to the cabinet subassemblies.  The copper traces for the DC voltages shall support at least 10 Amp.

- The circuit board shall be protected by an aluminum cover that includes the cabinet mounting flanges.

- The DC Power/Communication Bus shall be mounted on the left side wall of the cabinet.

(8) AC Clean Power Bus.

The AC Clean Power Bus shall include eight (8) NEMA 5-15 receptacles to provide AC Clean Power to the ATC Cabinet Assemblies, the Controller, and Cabinet Power Supply.

(9) Switch Shield.

- The cabinet shall include a technician switch shield.  This shield shall protect the police panel switch wiring and include technician test switches for Stop Time, Auto/Flash, and a 24 VDC Bypass Switch.  This 24 VDC Bypass switch shall be a momentary push button switch that, when pressed, energizes the 24 VDC to the HDSPs during Flash Mode.  The button shall be labeled “24 VDC BYPASS”.  This allows a technician to momentarily apply 24 VDC power to the HDSPs while in a Flash condition.

- The switch shield shall include a duplex receptacle with USB ports for device charging.  This receptacle shall be powered by the Raw AC Power circuit breaker.

(10) Police Panel.

- The cabinet shall include a police panel with switches for Auto/Flash, Signals On/Off, and Auto/Manual.  The Auto/Manual shall enable Manual Control Enable in the controller when in the Manual position.

- The cabinet shall include a hard-wired manual control cable with a 6 ft coiled cord.  This cable shall enable Interval Advance in the controller when depressed.

(11) Slide-out Drawer/Shelf.

- A telescoping slide-out drawer shall be provided for document storage.

- The Slide-Out Drawer/Shelf shall be mounted underneath the shelf and include a lip or handle for pulling.

- The drawer shall have a hinged top cover.

- The drawer shall be capable of accommodating one (1) complete set of cabinet prints and manuals.

- The drawer shall support 50 lb (23 kg) in weight when fully extended.

- The drawer shall open and close smoothly.

- The drawer dimensions shall make maximum use of available depth offered by the controller shelf and be a minimum of 18 in. (610mm) wide.

Add the following to Article 1074.03 of the Standard Specifications:

(c) ATC Cabinet Plugins.

(1) Model 2202-HV High-Density Switch Pack/Flasher Unit (HDSP/FU).

- The HDSP/FU shall be compact, pluggable, modular PCB-based, and equipped with a DIN connector.

- The HDSP/FU shall be compatible with ultra-low power LED signal heads and shall have a current monitoring feature for each output of each channel.

- The HDSP/FU shall use real-time standardized high speed SB3 communications with the Cabinet Monitor Unit to send a complete set of RMS voltage and load current measurements.

- The HDSP/FU shall be 4.5 in. H x 6.5 in. D and shall be equipped with a handle, reset push button switch, six RYG LED indictors, four flasher LED indicators, one power LED indicator, and two Rx/Tx LED indicators.

- The HDSP/FU can function as either a switch pack (HDSP) or as a flasher unit (HDFU).

  - When installed in the Output Assembly, the HDSP shall provide two RYG channels of operation (6 individual field outputs).

  - When installed in the Service Assembly, the HDFU shall function as a four output flasher.

(2) Model 2212-HV Cabinet Monitor Unit (CMUip).

- The Cabinet Monitor Unit (CMUip) shall be compact, pluggable and modular.

- The CMUip shall use real-time standardized 614.4 kbps SDLC communications with the ATC to transfer command and response data on Serial Bus #1 (SB1).

- The CMUip shall be capable of monitoring up to 32 physical switch pack channels (RYG) and shall have optional four virtual channels.

- The CMUip shall provide a Flasher Alarm feature.  This alarm shall not put the cabinet into a Flash condition.

- The CMUip shall analyze the ATC output commands and field input status to isolate the failure source by channel and color.

- The CMUip configuration programming shall be provided by an interchangeable Datakey nonvolatile memory device.  This rugged key shall store all CMUip configuration parameters and shall eliminate programming using jumpers, diodes, or DIP switches.

- The CMUip shall maintain a nonvolatile event log recording the complete intersection status as well as time stamped previous fault events, AC Line events, configuration changes, monitor resets, cabinet temperature and true RMS voltages and currents for all field inputs.

- The signal sequence history log stored in nonvolatile memory graphically shall display up to 30 seconds of signal status prior to the fault trigger event with 50 ms resolution to ease diagnosing of intermittent and transient faults.

(3) Model 2218 Serial Interface Unit (SIU).

- The Model 2218 Serial Interface Unit (SIU) shall be a compact, pluggable and modular.

- The SIU shall use real-time standardized 614.4 kbps SDLC communications with the ATC to transfer command and response data on Serial Bus #1 (SB1).

- The SIU shall be equipped with 54 programmable input/output pins, four (4) optically isolated input pins, one line sync reference input pin, and four (4) address select input pins. The optically isolated inputs shall work with either 12 VAC or 24 VDC.

- The SIU outputs shall be rated at 150 mA continuous sink current.  Each output shall provide a 500 mA typical current limit and shall be rated to 50 V and utilize a voltage clamp for inductive transient protection.

- The SIU shall be equipped with a front panel LED indicator that can report the current SIU assembly address assignment of the SIU for cabinet configuration verification.

- The SIU shall require a nominal supply voltage of 24 VDC (+/- 2 VDC).  A voltage of 16 VDC or less shall be considered loss of power, and a voltage of 18 VDC or greater shall be considered adequate for operation.  The SIU shall not require more than 300 mA over the voltage range of 16 VDC to 30 VDC and the power surge shall be limited to a maximum of 1.25 A from initial application of DC power.  The SIU shall not be damaged by insertion to, or removal from, powered input or output assemblies.  The SIU shall operate normally for 700 ms after power loss.

(4) Model 2220 Auxiliary Display Unit (ADU).

- The ADU shall install in a 1U height of the rack space and shall provide a menu driven user interface to the enhanced features of the CMUip monitor, including the built-in Diagnostic Wizard.

- The ADU shall provide 32 channels of Red, Yellow and Green LED indicators that display full intersection status and 32 Blue fault status LED indicators to identify faulty channels.

- The ADU shall provide proper electrical termination to SB3.

- The ADU shall have a 4 line by 20 character menu driven liquid crystal display with backlight and heater.

- The ADU built-in Diagnostic Wizard shall automatically pinpoint faulty signals, offer trouble-shooting guidance, and automatically isolate and identify problems.

- The ADU shall be equipped with Event Logging displaying the CMUip time-stamped nonvolatile event log records with the complete intersection status, as well as AC Line events, monitor resets, temperature and true RMS voltages and currents.

(5) Model 2216 Cabinet Power Supply (CPS).

- The Model 2216 Cabinet Power Supply is a modular 19 in. rack mounted power supply device providing the DC voltages necessary for operating the ATC Cabinet.  Power Factor Correction shall be provided.  The PS2216-24-HV provides a regulated 24 VDC output and a regulated 48 VDC output.  The PS2216-2412-HV model provides a regulated 24 VDC output, a regulated 48 VDC output, and a regulated 12 VDC output option for powering the Input Assembly devices.  Unless otherwise specified, all PS2216 requirements apply to both model types.

- The CPS shall be 1U in height maximum and designed to mount into a 19 in. EIA rack.  The maximum depth of the PS2216 shall be less than 8 in.

- The CPS shall be powered from AC Line provided by an AC Line cord with NEMA Type 515 plug.  The input voltage range shall be 80 to 270 VAC, 45 to 65 Hz.  Power Factor Correction shall be greater than 0.95.

- The PS2216-2412 shall provide:

  - 48 VDC +/- 2 VDC at 1 A maximum

  - 24 VDC +/- 2 VDC at 4 A maximum

  - 12 VDC +/- 1 VDC at 5 A maximum

- Each DC output shall be electrically isolated from AC Mains and Earth Ground.  The 24 VDC and 12 VDC outputs share a common ground.

- The DC Output Ripple on each output shall be less than 300 mVpp when measured at 20 MHz of bandwidth using a 12 in. twisted pair-wire terminated with a 0.1 µf & 47 µf capacitor.

- The DC outputs shall attain regulated output levels within 500 ms of applied AC Line voltage of 110 VAC nominal across the operating temperature range and at rated full load.

- The DC outputs shall maintain regulated output level for a minimum of 50 ms across the operating temperature range and at rated full load.

- All indicators shall be clear LEDs.  Clear LEDs shall not depend on a reflector or diffusion as part of its design.  Clear LEDs shall not appear to be ON when exposed to ambient light.  The following indicators shall be provided:

  - A green AC Line indicator shall illuminate to indicate Operational input voltage is proper and the AC Line fuse is intact.

  - A green indicator shall illuminate to indicate the 48 VDC output is active and the fuse is intact.

  - A green indicator shall illuminate to indicate the 24 VDC output is active and the fuse is intact.

  - A green indicator shall illuminate to indicate the 12 VDC output is active and the fuse is intact (PS2216-2412 only).

- Banana style test jacks shall be provided on the front panel for each DC output and DC ground.  Mating banana plug spring width shall be 0.175 in. Nominal.

- The output connector shall be a Phoenix Contact #1825161 and shall mate with a Phoenix Contact #1825352 or equivalent.  Pin #1 shall be the right most pin when viewed from the rear of the supply.

Table 2 – Power Supply Connector Pin-Out

| Pin | Function |
| --- | --- |
| 1 | +48VDC |
| 2 | 48VDC Ground** |
| 3 | +24VDC |
| 4 | +12VDC (PS2216-2412 only) |
| 5 | 24/12VDC Ground |
| 6 | Chassis Ground |

**The 48 VDC output shall be electrically isolated from the AC Line input and the 24VDC and 12 VDC outputs.  The 48VDC Ground (pin #2) must be connected within the cabinet to the same AC Neutral that the Cabinet Monitor Unit (CMU) is connected to in the cabinet.

(6) MonitorKey Programming Tool.

- The Programming Tool provides the capability to read and write data from the CMUip Datakey device.

- The MonitorKey software shall be compatible with the CMUip-2212.

(7) Model 21H High-Density Flash Transfer Relay (HDFTR).

- The HDFTR shall be a Struthers-Dunn model 21XBXHL-48VDC or approved equal.

- The HDFTR shall have a hermetically sealed cover and shall be moisture proof.

- The HDFTR shall be filled with dry nitrogen to protect contacts from corrosion and to prevent condensation.

- The HDFTR shall have a cupronickel cover that is salt water resistant.

- The HDFTR contacts shall be rated at 10 A @ 120 VAC.

- The coil of the HDFTR shall be rated at 48 VDC.

- The HDFTR shall have an LED indicator to display contact transfer status.

(8) Main Contactor (MC).

- The MC shall be a combination solid-state and electromechanical relay in parallel and shall be rated at 60 A @ 120 VAC and 50 A @ 240 VAC.  The coil of the MC shall be rated at 48 VDC.

- The MC shall be equipped with input indicator and shall have SPST- N.O. contacts.

- The MC shall be hermetically sealed.

- The MC shall be easily accessible without removing any covers or panels.

- The MC shall be mounted on the Service Assembly for access to the wiring and to view the LED indicator.

(9) Cabinet Suppressor-Filter.

- The cabinet shall be equipped with a pluggable Cabinet Suppressor–Filter mounted in the Service Assembly.  The pluggable Cabinet Suppressor-Filter shall be an Asco Power SHA-1250, Hesco HE1750, or approved equivalent.

- The unit shall incorporate the use of warning and failure indicators and shall have a dry relay contact remote sensing circuit.  The unit shall be modular and pluggable with a 12-position Beau 5412 connector.

- The unit shall be rated at continuous service current of 15 A and maximum clamp voltage of 390 VAC.  The unit shall filter noise and spike from 10 KHz to 25 MHz and shall have a peak surge current of 48 kA.

(10) Detection Module Suppressor.

- The Detection Module Suppressor shall be Hesco model HE6LC-6 or Asco Power model MRA-6LC-6.

- The Detection Module Suppressor shall be modular and pluggable.

- The unit shall be epoxy encapsulated and equipped with 6-position 5.08 mm Phoenix Contact or approved equal connector.

- The unit shall be able to protect 6 circuits.

- The device operating voltage shall be 75 VAC and clamping voltage shall be 130 VDC.

- The device dimensions shall be 2 in. H x 7/10 in. W x 1-1/5 in. D.

(11) HDSP Protector.

- The HDSP Protector shall be modular and pluggable.  The unit shall be a Hesco HE103C-9, Asco MPA303-9, or approved equal.

- The unit shall be epoxy encapsulated and equipped with 9-position 5.08 mm Phoenix Contact connector or approved equal.

- The unit shall be able to protect 6 circuits.

- The device operating voltage shall be 120 VAC and clamping voltage shall be 340 VAC.

- The unit dimensions shall be 2 in. H x 7/10 in. W x 2 in. D.

(d) Auxiliary Cabinet Equipment.

(1) Ventilation Fans.

- Two (2) thermostatically controlled fans shall be provided to ventilate the cabinet.

- The fans shall be equipped with ball or roller bearings and shall each have a minimum capacity of 100 cu ft of free air delivery per minute.

- The fans shall be protected by a finger guard.

- The fan circuit shall be protected at 125% of the fan motor ampacity.

(2) Heater.

One (1) 800 W, thermostatically controlled, electric heater with automatic fan function.

(3) Thermostat.

- The thermostat shall be manually adjustable between 80°F and 170°F with a differential of not more than 10°F between automatic turn on and off.

- The manual adjustment shall be graded in 10°F increment scale.

- The Thermostat shall be a Bud Industries TS-15-A or approved equal.

(4) Lighting.

- The cabinet shall include two (2) LED light panels to illuminate the interior of the cabinet.

- One LED panel shall be mounted on the fan panel, and the other shall be mounted underneath the lower shelf.

- Both LED panels shall illuminate whenever the cabinet door is opened.

- The LED panels shall be provided from an approved Vendor.

(5) Insulation.

- Minimum half inch polystyrene rigid board insulation with a minimum R-3 thermal resistance attached to the top of cabinet on the interior side.

Basis of Payment.

This work will be paid for at the Contract unit price each for ADVANCED TRANSPORTATION CONTROLLER AND TYPE IV STRETCHED CABINET; ADVANCED TRANSPORTATION CONTROLLER AND TYPE SUPER R CABINET; ADVANCED TRANSPORTATION CONTROLLER AND TYPE IV STRETCHED CABINET, SPECIAL; ADVANCED TRANSPORTATION CONTROLLER AND TYPE SUPER R CABINET (SPECIAL).