# STABILIZED MEDIAN SURFACE (D1)

Effective:  September 30, 1985

Revised:  March 2, 2017

This work consists of constructing stabilized median surface according to typical drawings and details included in the plans and Section 355 of the Standard Specifications, except that all references to "Base Course" in said specifications shall be interpreted to mean "Median Surface."  Hot-Mix Asphalt Surface Course, Mix D, of the N design specified on the plans meeting the requirements of Section 406 of the Standard Specifications shall be used for the top lift [2 inch (50 mm) nominal thickness].

Method of Measurement.  The median surface shall be measured according to the requirements of Article 355.10 of the Standard Specifications.

Basis of Payment.  This work will be paid for at the contract unit price per square yard (square meter) for STABILIZED MEDIAN SURFACE.

60626300		STABILIZED MEDIAN SURFACE	         	SQ YD

M6066400		STABILIZED MEDIAN SURFACE		SQ M

H:\SP\DES\6068965R.DOC