Fiber Optic Utility Marker

Effective:  January 1, 2026

Description.

This work shall consist of furnishing and installing a fiber optic utility marker post.

Materials.

Fiber optic utility marker posts shall be made of non-conductive high-density polyethylene with a minimum outside diameter of 3.5 inches. The length above grade shall be a minimum of 54 inches or as shown on the plans, whichever is greater. The minimum anchor length below grade shall be 18 inches.

IDOT marker posts shall be white in color with an orange cap with black graphic and lettering on two sides. Lettering on the IDOT markers shall read “IDOT 847-705-4611”

Third-Party markers shall be integrally orange in color with an orange cap with black graphic and lettering on two sides. Lettering on the Third-Party markers shall read “IDOT 847-705-4611”.

All colors shall be stabilized against ultraviolet light such that they will not fade under continuous exposure to direct sunlight. The marker shall retain dimensional stability in temperatures ranging between -40°F and 175°F. Each post shall be able to withstand a single vehicle impact at 45 MPH and return to within 10 degrees of vertical within 60 seconds.

Installation.

Fiber optic utility marker posts shall be installed along the path of the fiber optic raceway installation between access points at a maximum spacing of 500 feet and changes in direction. For sections where IDOT and Third Party microduct conduit are installed in a common trench, separate fiber-optic utility marker posts shall be installed, one for IDOT and one for the third party.

Method of Measurement.

This work will be measured for payment in units of each.

Basis of Payment.

This work will be paid for at the contract unit price per each for FIBER OPTIC UTILITY MARKER.