# SELECTIVE CLEARING (ACRE)

Effective:  October 29, 2024

Description:  This work shall consist of extensive removal and disposal of shrubs, brush, weed material, debris (including trash, bottles, etc.) and selected trees up to six (6) inches in diameter.  Selective clearing shall include removal of typical amounts of litter and debris encountered during tree removal operations.  All trees and shrubs to be saved shall be carefully protected as provided by Article 201.05 of the Standard Specifications.  Locations for Selective Clearing and vegetation to be cleared or saved shall be designated by the Roadside Development Unit.  Contractor shall contact a representative of the Roadside Development Unit at (847) 705-4171 at least 2 weeks prior to work.

Submittals:

Contractor personnel applying herbicides shall submit their current license to the Engineer.  The licensed pesticide applicator shall be qualified at a minimum in Right-of-Way and Aquatics.

Contractor shall provide the Engineer with a list of herbicides, surfactants, water conditioners, dyes, pH balancers, and other chemicals and adjuvants to be used for implementation of this project for prior approval.

Prior to commencement of any work, submit to the Engineer a written description of all mechanical equipment and its intended use during the execution of the work for prior approval.

Removal and Initial Cut Stump Treatment:

The undesirable trees and brush (i.e. Tree of Heaven, Callery Pear, Siberian Elm, European Buckthorn, Mulberry, Ash, Russian Olive, Eurasian Honeysuckle, etc.) shall be cut flush with the ground.  All stumps shall be cut flat with no sharp points, and less than two (2) inches of surrounding grade.

All stumps shall be treated with an approved resprout herbicide mixed with a marking dye within twenty-four (24) hours of the tree being cut to prevent regrowth from those stumps.  Resprout herbicide shall be included in the cost of SELECTIVE CLEARING.

The Engineer may specify certain areas as “HAND CLEAR ONLY” to be avoided by mechanical equipment or access paths.  In these areas, the Contractor is prohibited from using mechanical clearing equipment due to sensitive site conditions.

All herbicides shall be applied according to the manufacturer’s label specifications.  Contractor’s personnel applying the resprout herbicide shall have a valid pesticide applicator license issued by the Illinois Department of Agriculture.

Branches on remaining trees shall be pruned off up to 12 feet from the ground or as directed by the Engineer depending on tree species.

The resprout herbicide shall be approved by the Engineer.  Resprout herbicide shall be labeled to control woody species present within clearing areas. Resprout herbicide shall be included in the cost of SELECTIVE CLEARING.

The Contractor shall maintain copies at the project site of all current pesticide herbicide labels and Material Safety Data Sheets (MSDS) for all chemicals utilized during completion of the work.

Damages to existing vegetation to remain, such as broken limbs, or other plantings or roadside appurtenances caused by the Contractor’s tree removal or trimming operations shall be repaired at the Contractor’s expense to the satisfaction of the Engineer.

All cleared areas shall be graded, trimmed, smoothed, finished uniformly, and left ready to be seeded and blanketed to the satisfaction of the Engineer with equipment approved by the Engineer.  The ground shall be relatively free of trash, slash, and woody debris or other foreign material which will prevent the close contact of the hydromulch or blanket.  Disposal of material shall be done in accordance with Article 202.03.  The Engineer shall have the ultimate authority to approve the final condition of slash.  In areas where seeding will take place the use of a forestry mower to manage minor woody vegetation, grind slash, stumps under 6”, and any remaining woody plant debris down to the surface of the soil to prepare the site for future seeding may be needed.

Protection of soils from compaction, erosion, and disturbance are the Contractor’s responsibility prior to start of work.  Any damage caused by Contractor including but not limited to tire ruts, damage to turf, damage to drainage swales, damage to fence, damage to road pavement, etc. shall be repaired by the Contractor at the Contractor’s expense to the satisfaction of the Engineer.

Method of Measurement:  Selective Clearing will be measured in units of 1 ACRE.  The unit price shall include the cost of all material, equipment, labor, disposal, and incidental items required to complete the work as specified herein and to the satisfaction of the Engineer.

If the inspection discloses any work as being unsatisfactory, the Engineer will give the Contractor the necessary instructions for correction of same, and the Contractor shall immediately comply with such instructions and correct the unsatisfactory work.  Areas not meeting the satisfaction of the Engineer shall not be measured for payment.  Plan quantities are estimates only.  Actual quantities will be measured in place.  Agreement to plan quantities will not be allowed.

Basis of Payment:  This work will be paid for at the contract unit price per acre for SELECTIVE CLEARING.  Payment for Selective Clearing shall include the cost of all minor grading, slash/debris removal and disposal, trimming, pruning, smoothing, finishing, labor, materials, tools, and equipment required to complete the work as specified herein and to the satisfaction of the Engineer.