# ELECTRIC METER

Effective: November 1, 2023

805.02TS

Description.

This work shall consist of furnishing a ringless meter socket meeting the

requirements of the power company. The meter socket shall be installed on the side of the existing

traffic signal controller cabinet, opposite of the UPS side of the cabinet in accordance with the

details provided in the plans at existing unmetered traffic signal locations, or as directed by the

engineer.

Materials.

The meter socket shall meet the following requirements:

- CECHA Approved

- Single Position

- Number of Jaws = 4 Terminal

- Voltage rating of 600 Volts Alternating Current

- Amperage rating of 200 Continuous Ampere

Basis of Payment.

This item will be paid for at the contract unit price per each for ELECTRIC

METER. The unit price shall include all equipment, materials, and labor required to furnish, and

install the electric meter socket and related hardware components.