# LIGHT EMITTING DIODE (LED) SIGNAL HEAD AND OPTICALLY PROGRAMMED LED SIGNAL HEAD

Effective: May 22, 2002

Revised: March 1, 2025

880.01TS

Materials.

Add the following to Section 1078 of the Standard Specifications:

“LED modules proposed for use and not previously approved by IDOT District One will require independent testing for compliance to current VTCSH-ITE standards for the product and be Intertek ETL Verified.  This would include modules from new Vendors and new models from IDOT District One approved Vendors.

The proposed independent testing facility shall be approved by IDOT District One.  Independent testing must include a minimum of two (2) randomly selected modules of each type of module (i.e. ball, arrow, pedestrian, etc.) used in the District and include as a minimum Luminous Intensity and Chromaticity tests.  However, complete module performance verification testing may be required by the Engineer to assure the accuracy of the Vendor’s published data and previous test results.  An IDOT representative will select sample modules from the local warehouse and mark the modules for testing.  Independent test results shall meet current ITE standards and vendor’s published data.  Any module failures shall require retesting of the module type.  All costs associated with the selection of sample modules, testing, reporting, and retesting, if applicable, shall be the responsibility of the LED module Vendor and not be a cost to this Contract.

All signal heads shall provide 12 in. (300 mm) displays with glossy yellow or black polycarbonate housings.  All head housings shall be the same color (yellow or black) at the intersection.  For new signalized intersections and existing signalized intersections where all signals heads are being replaced, the proposed head housings shall be black.  Where only selected heads are being replaced, the proposed head housing color (yellow or black) shall match existing head housings.  Connecting hardware and mounting brackets shall be polycarbonate (black).  A corrosion resistant anti-seize lubricant shall be applied to all metallic mounting bracket joints and shall be visible to the inspector at the signal turn-on.  Post top mounting collars are required on all posts and shall be constructed of the same material as the brackets.

The LED signal modules shall be replaced or repaired if an LED signal module fails to function as intended due to workmanship or material defects.  LED signal modules which exhibit luminous intensities less than the minimum values specified in Table 1 of the ITE Vehicle Traffic Control Signal Heads: Light Emitting Diode (LED) Circular Signal Supplement (June 27, 2005) [VTCSH], or applicable successor ITE specifications, or show signs of entrance of moisture or contaminants, shall be replaced or repaired.  The Vendor’s written warranty for the LED signal modules shall be dated, signed by a Vendor’s representative, and included in the product submittal to the State.  See Article 801.14 of the Standard Specifications for warranty information.

- Physical and Mechanical Requirements

- Modules can be manufactured under this specification for the following faces:

- 12 in. (300 mm) circular, multi-section

- 12 in. (300 mm) arrow, multi-section

- The maximum weight of a module shall be 4 lb (1.8 kg).

- Each module shall be a sealed unit to include all parts necessary for operation (a printed circuit board, power supply, a lens and gasket, etc.) and shall be weatherproof after installation and connection.

- The lens of the module shall be tinted with a wavelength-matched color to reduce sun phantom effect and enhance on/off contrast.  The tinting shall be uniform across the lens face.  Polymeric lens shall provide a surface coating or chemical surface treatment applied to provide abrasion resistance.  The lens of the module shall be integral to the unit, convex with a smooth outer surface and made of plastic.  The lens shall have a textured surface to reduce glare.

- The use of tinting or other materials to enhance ON/OFF contrasts shall not affect chromaticity and shall be uniform across the face of the lens.

- Each module shall have a symbol of the type of module (i.e. circle, arrow, etc.) in the color of the module.  The symbol shall be 1 in. (25.4 mm) in diameter. Additionally, the color shall be written out in 1/2 in. (12.7mm) letters next to the symbol.

- Photometric Requirements

- The LEDs utilized in the modules shall be AlInGaP technology for red and InGaN for green and amber indications and shall be the ultra bright type rated for 100,000 hours of continuous operation from -40 °C to 74 °C.

- Electrical

- Maximum power consumption for LED modules as per the tables in Article 1078.01.

- Operating voltage of the modules shall be 120 VAC.  All parameters shall be measured at this voltage.

- The modules shall be operationally compatible with currently used controller assemblies (solid state load switches, flashers, and conflict monitors).

- When a current of 20 mA AC or less is applied to the unit, the voltage read across the two leads shall be 15 VAC or less.

- The LED modules shall provide constant light output under power.  Modules with dimming capabilities shall have the option disabled or set on a non-dimming operation.

- LED arrows shall be wired such that a loss or the failure of one or more LEDs will not result in the loss of the entire module.

- Retrofit Traffic Signal Module

The following specification requirements apply to the Retrofit module only.  All general specifications apply unless specifically superseded in this section.

- Retrofit modules can be manufactured under this specification for the following faces:

- 12 in. (300 mm) circular, multi-section

- 12 in. (300 mm) arrow, multi-section

- Each Retrofit module shall be designed to be installed in the doorframe of a standard traffic signal housing.  The Retrofit module shall be sealed in the doorframe with a one-piece EPDM (ethylene propylene rubber) gasket.

- The maximum weight of a Retrofit module shall be 4 lb (1.8 kg).

- Each Retrofit module shall be a sealed unit to include all parts necessary for operation (a printed circuit board, power supply, a lens and gasket, etc.) and shall be weatherproof after installation and connection.

- Electrical conductors for modules, including Retrofit modules, shall be 39-2/5 in. (1 m) in length, with quick disconnect terminals attached.

- The lens of the Retrofit module shall be integral to the unit, shall be convex with a smooth outer surface and made of plastic or of glass.

- The following specification requirements apply to the 12 in. (300 mm) arrow module only.  All general specifications apply unless specifically superseded in this section.

- The arrow module shall meet specifications stated in Section 9.01 of the Equipment and Material Standards of the Institute of Transportation Engineers (November 1998) [ITE Standards], Chapter 2 (Vehicle Traffic Control Signal Heads) or applicable successor ITE specifications for arrow indications.

- The LEDs arrow indication shall be a solid display with a minimum of three (3) outlining rows of LEDs and at least one (1) fill row of LEDs.

- The following specification requirement applies to the 12 in. (300 mm) programmed visibility (PV) module only.  All general specifications apply unless specifically superseded in this section.

- The LED module shall be a module designed and constructed to be installed in a programmed visibility (PV) signal housing without modification to the housing.

Delete the fourth paragraph of Article 880.03 of the Standard Specifications.  Refer to the “Bagging Signal Heads” section of the District 1 Traffic Signal Special Provision 800.01TS TRAFFIC SIGNAL GENERAL REQUIREMENTS.”

Basis of Payment.

Add the following to the first paragraph of Article 880.04 of the Standard Specifications:

“The price shall include furnishing the equipment described above, all mounting hardware and installing them in satisfactory operating condition.”

Revise the second paragraph of Article 880.04 of the Standard Specifications to read:

If the work consists of retrofitting an existing polycarbonate traffic signal head with light emitting diodes (LEDs), it will be paid for as a SIGNAL HEAD, LED, RETROFIT, of the type specified, and of the particular kind of material, when specified.  Price shall be payment in full for removal of the existing module, furnishing the equipment described above including LED modules, all mounting hardware, and installing them in satisfactory operating condition.  The type specified will indicate the number of signal faces, the number of signal sections in each signal face and the method of mounting.