# TRAFFIC SIGNAL BACKPLATE, LOUVERED, RETROREFLECTIVE

Effective: March 1, 2026

882.02TS

Description.  This work shall consist of furnishing and installing traffic signal backplates in accordance with Section 882 of the Standard Specifications, except as modified herein.

Materials.

Revise the first paragraph of Article 1078.03 of the Standard Specifications to read:

“The traffic signal backplate shall be made of composite laminated aluminum or ABS plastic (vacuum formed). The composite laminated aluminum shall have a nominal thickness of 0.080 in. (2.0 mm) with a nominal 0.01 in. (0.3 mm) aluminum skin on both sheet sides and shall have a minimum tensile strength of 4,300 psi at 73 °F (30,000 kPa at 23 °C). The vacuum formed ABS plastic backplate shall have a nominal thickness of 1/8 in. (3 mm), a nominal 1/2 in. (13 mm) deep back flange on all inside and outside edges, and a minimum tensile strength of 4,300 psi at 73 °F (30,000 kPa at 23 °C).”

Revise the first sentence of the second paragraph of Article 1078.03 of the Standard Specifications to read:

“The backplate shall be composed of one or two pieces.”

Revise the third paragraph of Article 1078.03 of the Standard Specifications to read:

“The backplate shall have a retroreflective sheeting.  Retroreflective sheeting shall be Type ZZ sheeting according to Article 1091.03 and applied in preferred orientation for the maximum angularity according to the vendor’s recommendations.  The retroreflective sheeting shall be installed under a controlled environment by the Manufacturer/Vendor before shipment to the Contractor.  The formed plastic backplate shall be prepared and cleaned, following recommendations of the retroreflective sheeting Manufacturer.”

Revise the fourth paragraph of Article 1078.03 of the Standard Specifications to read:

“The traffic signal backplate shall be louvered to provide openings (louvers) to allow wind to penetrate and reduce wind loading.  Retroreflective sheeting shall not interfere with the louvers.”

Basis of Payment.  This work shall be paid for at the contract unit price per each for TRAFFIC SIGNAL BACKPLATE, LOUVERED, RETRORELECTIVE.