# DETECTOR LOOP

Effective: May 22, 2002

Revised: March 1, 2024

886.01TS

Procedure.

A minimum of seven (7) working days prior to the Contractor cutting loops, the Contractor shall mark the proposed loop locations and contact the Area Traffic Signal Maintenance and Operations Engineer to inspect and approve the layout.  When preformed detector loops are installed, the Contractor shall have them inspected and approved prior to the pouring of the Portland cement concrete surface using the same notification process as above.

Installation.

Revise Article 886.04 of the Standard Specifications to read:

“Loop detectors shall be installed according to the requirements of the “District One Standard Traffic Signal Design Details.”  Saw-cuts (homeruns on preformed detector loops) from the loop to the edge of pavement shall be made perpendicular to the edge of pavement when possible in order to minimize the length of the saw-cut (homerun on preformed detector loops) unless directed otherwise by the Engineer or as shown on the plans.

The detector loop cable insulation shall be labeled with the cable specifications.

Each loop detector lead-in wire shall be labeled in the handhole using a waterproof tag secured to each wire with nylon ties.

Resistance to ground shall be a minimum of 500 mega-ohms under any conditions of weather or moisture.  Inductance shall be more than 50 and less than 700 microhenries.

- Type I.  All loops installed in new asphalt pavement shall be installed in the binder course and not in the surface course.  The edge of pavement, curb, and handhole shall be cut with a 1/4 in. (6.3 mm) deep x 4 in. (100 mm) saw cut to mark the location of each loop cable.

- Loop sealant shall be two-component thixotropic chemically cured polyurethane from an approved Vendor.  The sealant shall be installed 1/8 in. (3 mm) below the pavement surface.  If installed above the surface, the excess shall be removed immediately.

- Preformed.  This work shall consist of furnishing and installing a rubberized or cross-linked polyethylene heat resistant preformed traffic signal loop in accordance with the Standard Specifications, except for the following:

  - Preformed detector loops shall be installed in the sub-base under the Portland cement concrete pavement.  Loop lead-ins shall be extended to a temporary protective enclosure near the proposed handhole location.  The protective enclosure shall provide sufficient protection from other construction activities and may be buried for additional protection.

  - Handholes shall be placed next to the shoulder or back of curb when preformed detector loops enter the handhole.  CNC, included in this pay item, shall be used to protect the preformed lead-ins from back of curb to the handhole.

  - Preformed detector loops shall be factory assembled with ends capped and sealed against moisture and other contaminants.  The loop configurations and homerun lengths shall be assembled for the specific application.  The loop and homerun shall be constructed using a minimum 5/8 in. (16 mm) outside diameter, minimum 3/8 in. (9.5 mm) inside diameter Class A oil resistant synthetic cord reinforced hydraulic hose with 250 psi (1,720 kPa) internal pressure rating or a similarly sized XLPE cable jacket.  The hose for the loop and homerun assembly shall be one continuous piece.  No joints or splices shall be allowed in the hose except where necessary to connect homeruns to the loops.  This will provide maximum wire protection and loop system strength.  Hose tee connections shall be heavy duty high temperature synthetic rubber.  The tee shall be of proper size to attach directly to the hose, minimizing glue joints.  The tee shall have the same flexible properties as the hose to ensure that the whole assembly can conform to pavement movement and shifting without cracking or breaking.  For XLPE jacketed preformed loops, all splice connections shall be soldered, sealed, and tested before being sealed in a high impact glass impregnated plastic splice enclosure.  The wire used shall be #16 THWN stranded copper.  The number of turns in the loop shall be application specific.  Homerun wire pairs shall be twisted a minimum of eight turns per foot.  No wire splices will be allowed in the preformed loop assembly.  The loop and homeruns shall be filled and sealed with a flexible sealant to ensure complete moisture blockage and further protect the wire.  The preformed loops shall be constructed to allow a minimum of 6-1/2 ft of extra cable in the handhole.”

Method of Measurement.

Add the following to Article 886.05 of the Standard Specifications:

“Preformed detector loops will be measured along the detector loop embedded in the pavement rather than the actual length of the wire.  Detector loop measurements shall include the saw cut and the length of the detector loop wire to the edge of pavement.  The detector loop wire, including all necessary connections for proper operations, from the edge of pavement to the handhole, shall be included in the price of the detector loop.  CNC, trench and backfill, and drilling of pavement or handholes shall be included in detector loop quantities.”

Basis of Payment.

This work shall be paid for at the contract unit price per foot (meter) for DETECTOR LOOP, TYPE I or PREFORMED DETECTOR LOOP as specified in the plans, which price shall be payment in full for furnishing and installing the detector loop and all related connections for proper operation.