# PEDESTRIAN PUSH-BUTTON

Effective: May 22, 2002

Revised: March 1, 2024

888.01TS

Description.

Revise Article 888.01 of the Standard Specifications to read:

“This work shall consist of furnishing and installing a latching (single call) or non-latching (dual call) pedestrian push-button and a regulatory pedestrian instruction sign according to MUTCD, sign series R10-3e 9 in. x 15 in. sign with arrow(s) for a countdown pedestrian signal.”

Installation.

Add the following to Article 888.03 of the Standard Specifications:

“A mounting bracket and/or extension shall be used to assure proper orientation when two pedestrian push buttons are required for one post.  The price of the bracket and/or extension shall be included in the cost of the pedestrian push button.  The Contractor is not allowed to install a push-button assembly with the sign below the push-button in order to meet mounting requirements.”

Materials.

Revise Article 1074.02(a) of the Standard Specifications to read:

“The pedestrian push-button housing shall be constructed of aluminum alloy according to ASTM B 308 6061-T6 and powder coated yellow, unless otherwise noted on the plans.  The housing shall be furnished with suitable mounting hardware.”

Add the following to Article 1074.02 of the Standard Specifications:

- Station.  Stations shall be designed to be mounted to a post, mast arm pole or wood pole.  The station shall be aluminum and shall accept a 3 inch (75mm) round push-button assembly and a regulatory pedestrian instruction sign according to MUTCD, sign series R10-3e 9” x 15” sign with arrow(s) for a count-down pedestrian signal.  The pedestrian station size without count-down pedestrian signals shall accommodate a MUTCD sign series R10-3b or R10-3d 9” x 12” sign with arrow(s).

- Location.  Pedestrian push-buttons and stations shall be mounted to a post, mast arm pole, or wood pole as shown on the plans and shall be fully ADA accessible from a paved or concrete surface.  See the District’s Detail sheets for orientation and mounting details.

Basis of Payment.

Revise Article 888.04 of the Standard Specifications to read:

“This work will be paid for at the Contract unit price per each for PEDESTRIAN PUSH-BUTTON or PEDESTRIAN PUSH-BUTTON, NON-LATCHING.”