# ILLUMINATED SIGN, LED

Effective: May 22, 2002

Revised: July 1, 2015

891.01TS

Revise the second paragraph of Article 1084.01(a) to read:

The exterior surface of the housing shall be acid-etched and shop painted with one coat of zinc-chromate primer and two coats of exterior enamel.  The housing shall be the same color (yellow or black) to match the existing or proposed signal heads.  The painting shall be according to Section 851 of the Standard Specifications.

Add the following to Article 1084.01 (b) of the Standard Specifications:

The message shall be formed by rows of LEDs.  The sign face shall be 24 inches (600 mm) by 24 inches (600 mm).

Revise Article 1084.01(d) to read:

Mounting hardware shall be black polycarbonate or galvanized steel and similar to mounting Signal Head hardware and bracket specified herein and shall provide tool free access to the interior.