# tERMINAL SERVER

Effective: November 1, 2023

892.02TS

Description.

This work shall consist of furnishing and installing a terminal server used to transmit

signal controller data from one or more traffic signal controllers onto the District 1 Advanced Traffic Management System Ethernet network. The Contractor shall furnish and install the required hardware at the location shown on the plans and/or as directed by the Traffic Engineer.

General.

The terminal server shall be one of the following:

• Digi PortServer TS Hcc 4 four-port serial-to-Ethernet device with 120V

power supply and Digi RJ45/DB25-male-DCE-48” cable

• Comtrol DeviceMaster DM-2304 four-port serial-to-Ethernet device with

120V power supply and a 9-pin to 25-pin serial cable

The Contractor shall provide a null modem if required by the manufacturer for communication.

The terminal server shall have anonymous FTP capabilities disabled by the vendor/equipment

supplier or provide a feature for the user to disable the functionality through the standard device

menus.

The terminal server shall be properly configured for its location within the District 1 Traffic

Ethernet Network, and for proper communication with the signal equipment being connected to it.

Except where indicated otherwise in the special provisions or plans, District 1 Traffic will provide the IP address and serial drop addresses upon request.

The required programming shall be included in the cost of this pay item.

Basis of Payment.

This work shall be paid for at the contract unit price per each for TERMINAL SERVER. The unit price shall include all equipment; materials; licenses, programming; testing and documentation; and labor required to add a traffic signal controller to the centralized system.