Regional Engineers

Jack A. Elston

Special Provision for Building Removal with Asbestos Abatement

April 22, 2022

This special provision was developed by the Bureau of Design and Environment to replace three BDE Special Provisions:  Building Removal – Case I (Non-Friable and Friable Asbestos), Building Removal – Case II (Non-Friable Asbestos), and Building Removal – Case III (Friable Asbestos).  This special provision combines the three into one, provides clarification on when non-friable asbestos cannot remain in the building during demolition, removes redundant statements, and updates to current practice.

It should be included on contracts involving building removal with friable and/or non-friable asbestos.  This special should be used when you have multiple buildings to be removed and at least one has asbestos.

Designer Note.  Include the following from the asbestos containing building material (ACBM) building inspection report:  (1) Sketches indicating the location of ACBMs, (2) Materials Description Table for a brief description and location of the various materials, and (3) Materials Quantities Table listing the approximate quantity of each friable and/or non-friable ACBM.  In identifying the buildings (on page 1 of this BDE Special Provision), include if asbestos has been found within the description.

The pay item BUILDING REMOVAL NO. ___ should be inserted for each building on the contract, regardless of asbestos type.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the August 5, 2022 and subsequent lettings.  The Project Coordination and Implementation Section will include a copy in the contract.

5026im

# BUILDING REMOVAL with Asbestos ABATEMENT (Bde)

Effective: September 1, 1990

Revised:  August 1, 2022

Description.  This work shall consist of the removal and disposal of building(s), including all foundations, retaining walls, and piers, down to a plane 1 ft (300 mm) below the ultimate bottom of building elevation or proposed bottom of construction elevation.  The building(s) are identified as follows:

Parcel

Bldg. No.		   No.   			Location			Description

CONSTRUCTION REQUIREMENTS

General.  The IEPA’s "State of Illinois Demolition/Renovation/Asbestos Project Notification Form” shall be submitted and a copy sent to the Engineer.  It shall be updated if there is a change in the start and/or finish date or if the quantity of asbestos changes by more than 20 percent.

Asbestos abatement work shall be performed by an IDPH licensed Contractor prequalified with the Illinois Capital Development Board who has an on-site supervisor licensed by IDPH and employs workers licensed by IDPH.  This work shall be completed according to the requirements of the U.S. Environmental Protection Agency (USEPA), IEPA, OSHA, and local regulatory agencies.

Discontinuance of Utilities.  The Contractor shall arrange for the discontinuance of all utility services and the removal of the metering devices that serve the building(s) according to the respective requirements and regulations of the city, county, or utility companies involved.  The Contractor shall disconnect and seal the service outlets.

Posting.  Upon execution of the contract and prior to the removal of any buildings, the Contractor shall paint or stencil, in contrasting colors of an oil base paint, on all sides of each building or structure, the following posting:

NO TRESPASSING

VIOLATORS WILL BE PROSECUTED

The postings shall be positioned prominently on the structure(s) so they can be easily read and at a sufficient height to prevent defacing.

Asbestos Abatement.  Friable asbestos containing building materials (ACBMs) and Category II non-friable ACBMs shall be removed from the building(s) prior to demolition.  Category II non-friable ACBMs include asbestos containing transite boards, siding, and other cementitious materials (cement pipe or highly weathered roofing shingles/materials) which have a likelihood of becoming friable during typical demolition activities (by crumbling, pulverizing, or otherwise reducing to powder) making them regulated asbestos containing materials (RACM).  Removed ACBM shall be kept separate from non-ACBM demolition debris for purposes of transport and disposal.

Category I non-friable ACBM may be kept in place for demolition or removal of the building unless it has become friable as determined by the ACBM inspector.  If the Contractor demolishes the building(s) with the non-friable asbestos in place, the following shall apply.

(a)	The Contractor shall continuously wet the non-friable ACBM and other building debris with water during demolition and loading for disposal.

(b)	The Contractor shall dispose of all demolition debris as ACBM.

The Contractor shall perform air monitoring during asbestos abatement activities.  Air sampling shall be conducted by a qualified air sampling professional.  Air sampling shall be conducted according to NIOSH Method 7400.  Air monitoring equipment shall be calibrated and maintained in proper operating condition.  The Contractor shall submit a copy of the air sampling professional’s certificate to the Engineer.  The results of the tests, and daily calibration and maintenance records shall be kept on site and be available to the Engineer upon request.

Personal monitoring shall be conducted per applicable OSHA regulations.  Excursion limits shall be monitored daily, and corrective actions taken immediately to bring excursions within OSHA permissible exposure limits.

When asbestos is removed prior to demolition, clearance testing per IDPH shall be conducted upon the removal of ACBM.

Submittals.  The following submittals shall be made to the Engineer prior to the start of the asbestos abatement:

(a)	Manufacturer's certification stating that vacuums, ventilation equipment, and other equipment required to contain airborne fibers conform to ANSI 29.2.

(b)	A listing of the brand name, manufacturer, and specification of all sealants or surfactants to be used.

(c)	Proof that arrangements for transport and disposal of ACBMs have been obtained (i.e., a letter of authorization to utilize designated landfill).

(d)	A detailed work plan of the Contractor’s anticipated procedures including the location and layout of decontamination units, the sequencing of work, the respiratory protection plan, a site safety plan, a disposal plan, and a detailed description of the methods to be used to control pollution.

(e)	Proof of the Contractor’s prequalification with Capital Development Board and employee certifications with IDPH.

Submittals that shall be made upon completion of abatement work:

(f)	Copies of waste chain-of-custodies, trip tickets, shipping manifests, or disposal receipts for asbestos waste materials removed from the work area.

(g)	Copies of each day’s work site entry logbook with information on worker and visitor access.

(h)	Logs documenting filter changes on respirators, HEPA vacuums, negative pressure ventilation units, and other engineering controls.

(i)	Test results of any bulk material analysis and air sampling data collected during the abatement including results of any on-site testing by any federal, state, or local agency.

Any holes, such as basements, shall be backfilled according to Article 502.10.

Basis of Payment.  This work will be paid for at the contract lump sum unit price for BUILDING REMOVAL NO. ___.

Removal and disposal of friable ACBM will be paid for at the contract lump sum unit price for REMOVAL AND DISPOSAL OF FRIABLE ASBESTOS, BUILDING NO. ____.

Removal and disposal of non-friable ACBM will be paid for at the contract lump sum unit price for REMOVAL AND DISPOSAL OF NON-FRIABLE ASBESTOS, BUILDING NO. ____.

5026I