Regional Engineers

Jack A. Elston

Special Provision for Pavement Marking

January 10, 2025

This special provision was developed by the Bureau of Operations to clarify that paint pavement markings will not be subject to a winter performance period inspection.  It has been revised to allow grooving to be installed to the shape of the letters and symbols, fix a typo from the 2022 Standard Specifications, and rename the BDE Special Provision from “Pavement Marking Inspection”.

This special provision should be inserted into all contracts with pavement markings.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 7, 2025 and subsequent lettings.  The Project Coordination and Implementation Section will include a copy in the contract.

80464m

# PAVEMENT MARKING (bde)

Effective:  April 1, 2025

Revised:  November 1, 2025

Revise the fourth sentence of the fourth paragraph of Article 780.05 of the Standard Specifications to read:

“	Grooves for letters and symbols shall be cut in a rectangular shape or in the shape of the proposed marking so the entire marking will fit within the limits of the grooved area.”

Revise the last sentence of the third paragraph of Article 780.08 of the Standard Specifications to read:

“	The Contractor shall install the preformed plastic pavement markings according to the manufacturer’s recommendations.”

Revise the second sentence of the first paragraph of Article 780.13 of the Standard Specifications to read:

“	In addition, thermoplastic, preformed plastic, epoxy, preformed thermoplastic, polyurea, and modified urethane pavement markings will be inspected following a winter performance period that extends from November 15 to April 1 of the next year.”

80464